Activation Block Architectural Trade-Offs

Introduction

The objective of this document is to define architectures and explore the trade-offs between them for the Activation Block. This block requires a memory-based Look-Up Table (LUT) implementation and synthesizable logic to support complex non-linear functions. We have defined five potential architectural topologies for evaluation.

Architecture Options

This section details the architectural topologies evaluated for the Activation Block. In all configurations, the block is designed to support 6 distinct activation functions. Furthermore, to minimize dynamic power consumption, every configuration incorporates a global enable mechanism on its memory decoder. This ensures the memory array is only accessed and evaluated when a memory-based non-linear function is actively requested, preventing unnecessary toggling and power waste.

- Option A: SRAM (Flexible RAM)

This option utilizes a single SRAM block (256 rows × 8 bits). The RAM serves as a fully programmable LUT. Unlike the ROM options, the content is not fixed at fabrication; the user can load any arbitrary activation function into the memory, allowing for post-silicon updates or support for custom functions.

Trade-off: This architecture offers maximum flexibility. However, because it relies on a standard 6-transistor bitcell, it consumes significantly more area per bit than the ROM alternatives. It also suffers from high static power due to leakage across all rows and requires moderate dynamic power to precharge the differential bitlines during reads.

- Option B: ROM – Unified Memory

This option utilizes a single, monolithic ROM block (1024 rows × 8 bits) to store the four non-linear functions: Tanh, Sigmoid, GELU, and SiLU. To optimize memory usage, ReLU and Identity are not stored in the ROM. Instead, they are implemented via a parallel synthesizable logic path. The specific function is accessed by concatenating the activation selection bits with the input data to form the address in the unified block.

Trade-off: This setup avoids the complex output routing and multiplexing required by the banked architectures. However, it requires a complex 10-to-1024 decoder, which creates a massive area overhead. Furthermore, the long 1024-cell bitlines create high parasitic capacitance, making this option slower and more power-hungry during active reads compared to the split configurations.

- Option C: ROM Banked Memory Family

This approach physically divides the memory into four independent banks, where each bank is 256 rows by 8 bits. Each bank is dedicated to one of the four non-linear functions: Tanh, Sigmoid, GELU, and SiLU. Just like the Unified option, ReLU and Identity are offloaded to synthesizable logic.

Building on the general enable mechanism we established for the block, this architecture utilizes the activation selection signal as a specific enable mechanism for the individual banks. This ensures that only the specific ROM bank corresponding to the chosen activation is enabled. Because only the relevant bank is active, only the bitlines of that specific bank are charged and driven.

Option C1: Shared Decoder (Area Optimized)

To minimize area overhead, a global decoder is shared across all four banks. The decoded wordline is physically broadcasted to every bank.

Tradeoff: This configuration minimizes logic area and routing complexity. However, it consumes more dynamic power. Even though we only evaluate the bitlines of the active bank, broadcasting the wordline toggles the access transistors in the inactive banks. This wastes power on parasitic gate capacitance.

Option C2: Independent Decoders (Power Optimized)

Instead of a global decoder, this configuration duplicates the decoding logic so each bank gets its own dedicated local decoder. Building on the block's general enable mechanism, this setup uses the activation selection signal to activate only the specific local decoder required for the chosen function.

Trade-off: This completely isolates the inactive banks. Since the disabled decoders stay quiet, no wordlines toggle in the inactive banks, which heavily reduces dynamic power. The downside is a larger physical footprint due to the duplicated logic.

- Option D: Logic-Synthesized LUT (Combinational Implementation)

Since each activation function requires only approximately 20 to 30 unique lookup entries, the synthesis tool does not need to infer a memory macro. Instead, Fusion Compiler maps the behavioral case statement directly to a combinational cone of standard cells, where each output value is hardwired into the gate-level netlist as fixed connections to VDD or VSS. This eliminates all memory peripheral overhead: no bitcells, decoders, sense amplifiers, precharge circuits, or bitlines.

Trade-off: This achieves the smallest area and lowest power for small tables, but the values are fixed at synthesis time and cannot be reprogrammed post-silicon. This approach only remains efficient for small table sizes; for hundreds or thousands of entries, a memory-based structure would be more area-efficient.

PPA and Complexity Summary

| Memory Type/ Feature | Option A: SRAM | Option B: ROM (Unified) | Option C: Rom Banked Memory Family | Option C2: Split ROM (Independent Decoders) |
| --- | --- | --- | --- | --- |
| Configuration | Single Block (256 rows × 8 bits) | Single Block (1024 rows × 8 bits) | 4 Small Banks (4 × [256 rows × 8 bits]) | Combinational Logic (No memory structure) |
| Est. Transistors Count | ≈ 17,600 | ≈ 29,000 | C1: ≈ 14,000 C2: ≈ 21,500 | Lowest (to be confirmed by synthesis) |
| Transistors per Bit | ≈ 8.6 (expensive) | ≈ 3.5 (moderate) | C1: ≈ 1.7 (highly efficient) C2 :≈ 2.6 (efficient) | N/A (no bitcell structure) |
| Decoder | Standard 8-to-256 | Complex 10-to-1024 | C1: Shared 8-to-256 C2: Four Independent 8-to-256 | None (address decoded implicitly by logic gates) |
| Dynamic Power (Read Only) | Moderate (Requires differential precharge of two bitlines per bit) | Medium-High (High parasitic capacitance due to long 1024-cell bitlines) | C1: Low-Moderate (Bitlines isolated, but wordlines toggle in all banks wasting power) C2: Lowest (Both wordlines and bitlines toggle only in the active bank) | Lowest (no bitline or wordline charging; only combinational switching on active path) |
| Static Power | High (leakage from 256 rows) | Low | C1: Low C2: Low (Slightly higher leakage than C1 due to extra decoders) | Negligible (no memory array leakage) |
| Delay | Very Fast (uses sense amplifier) | Slow (high bitline capacitance) | C1: Fast (low bitline capacitance) C2: Fast (low bitline capacitance, minimal prefix logic delay) | Very Fast (shallow combinational path through a few gate levels) |

Table 1. PPA and Complexity summary.

PPA Table

| Metric | Option A: SRAM | Option B: ROM (Unified) | Option C1: Split ROM (Shared Decoder) | Option C2: Split ROM (Independent Decoders) | Option D: Synthesized Logic (Selected) |
| --- | --- | --- | --- | --- | --- |
| Configuration | Single Block (256 × 8 bits) | Single Block (1024 × 8 bits) | 4 Banks (4 × [256 × 8 bits]) Shared Decoder | 4 Banks (4 × [256 × 8 bits]) Independent Decoders | Combinational standard cells (no memory structure) |
| Area | 1523.86 μm2 |  |  |  | 320.42 μm2 |
| Critical Path | 0.5943 ns |  |  |  | 0.74 ns |
| Dynamic Power | 286.07 μW (read only) |  |  |  | 15.6 μW |
| Static Power | 87.2 μW |  |  |  | 63 nW |

Table 1. Post-synthesis PPA comparison. The Option D area figure represents the sum of all four individually synthesized activation functions, serving as an upper bound. The power and critical path figures for Option D represent the worst case across the four functions.

Transistor Count Analysis

The estimated transistor counts presented in Table 1 represent the aggregate of the storage core (bitcells) and the peripheral circuitry (overhead). The following breakdown details the methodology used to derive these estimates for each architecture.

Option A: SRAM (Single Block: 256 rows × 8 bits)

- Total Estimate: ~17,600 transistors

- Core (6T Bitcell): 12,288 (256 rows × 8 bits × 6 transistors/cell)

- Overhead: ~5,300

Includes: 8-to-256 Row Decoder, differential sense amplifiers, write drivers, and precharge circuits.

Option B: ROM – Unified Memory (Single Block: 1024 rows × 8 bits)

- Total Estimate: ~29,000 transistors

- Core (1T Bitcell): 8,192 (1024 rows × 8 bits × 1 transistor/cell)

- Overhead: ~20,800

Includes: Complex 10-to-1024 decoder and 1,024 individual wordline drivers. The overhead scales poorly, becoming roughly 2.5x larger than the core itself.

Option C: ROM – Banked Memory Family (4 Banks × 256 rows × 8 bits)

- Core (1T Bitcell): 8,192 (Equivalent capacity to Option B, spread across 4 banks).

- Option C1: Shared Decoder (Area Optimized)

  - Total Estimate: ~14,000 transistors

  - Overhead: ~5,800

Includes: A single shared 8-to-256 decoder, bank select multiplexers, and basic enable logic.

- Option C2: Independent Decoders (Power Optimized)

  - Total Estimate: ~21,500 transistors

  - Overhead: ~13,300

Includes: Four dedicated 8-to-256 decoders (adding roughly 7,500 transistors to the baseline overhead), prefix enable logic, and bank select multiplexers.

Option D: Logic-Synthesized LUT

The transistor count methodology used for Options A through C does not apply to this architecture, as no memory structure exists. There are no bitcells, decoders, sense amplifiers, or precharge circuits to count. Instead, the synthesis tool generates a combinational logic cone whose size depends on the number of table entries and the complexity of the output bit patterns. For approximately 20 to 30 entries per function, this is expected to yield a transistor count significantly lower than all memory-based options. The exact count will be reported from the Fusion Compiler post-synthesis area report.

Physical Routing Trade-off

While logic area is minimized in the banked configurations, distributing the address bus to four separate macros and routing distinct output buses to a central multiplexer consumes additional metal layer area compared to a monolithic block. Option C2 adds further floorplan complexity due to the placement of the duplicated decoders.

Our Choice

We selected the logic-synthesized LUT (Option D). Since each activation function maps only approximately 20 to 30 unique entries, a full memory macro is unnecessary. The synthesis tool implements the entire table as combinational logic, eliminating all memory overhead at a fraction of the area and power cost. ReLU and Identity remain implemented via synthesizable logic, as in Options B and C.

Appendix: Activation Functions

| Activation Function | Logical Role & Behavior | Critical Application | Risk of Exclusion |
| --- | --- | --- | --- |
| ReLU (Rectified Linear Unit) | Pass-through / Zeroing | Computer Vision: The standard for image recognition (e.g., smart security sensors). Computationally efficient for deep networks. | PE would be inefficient and slow for standard image tasks. |
| Identity | Pass-through | Residual Learning: Crucial for "Skip Connections" in deep networks (e.g., ResNet) to prevent vanishing gradients, and for Linear Output Layers in regression tasks. | The PE would fail to support modern deep architectures (ResNets) or regression models, severely limiting its flexibility and accuracy on complex tasks. |
| Tanh (Hyperbolic Tangent) | Zero-Centered Mapping | Voice Processing: Essential for the "Cell State" in recurrent networks used in voice activation (e.g., smart toys). | PE would struggle to maintain long-term memory, failing at sequence tasks. |
| Sigmoid | Probability Switch | Required for "Forget/Update Gates". Decides what information to keep. | PE could not run standard LSTMs or GRUs without Sigmoid, breaking voice/audio applications. |
| GELU (Gaussian Error Linear Unit) | Probabilistic Smoothing | Modern AI (Transformers) | PE would be stuck supporting only older (pre-2018) models |
| SiLU (Sigmoid Weighted Linear Unit) | Self-Gating, allows small negative values to propagate | The standard activation for MobileNetV3 and EfficientNet, which are specifically designed for low-power edge devices | PE could not natively accelerate the most popular "mobile-optimized" networks, forcing them to fallback to less accurate approximations. |

Table 2. Activation functions.