Architectural Trade-Offs: Internal FSM vs. RISC-like Control for RESCALE Operation

Introduction

This document evaluates two different architectural approaches for implementing the RESCALE operation within the PE. The RESCALE function requires a uint x int multiplication followed by a right shift. Because a dedicated, single-cycle uint x int multiplier consumes too much area, we need an efficient alternative that reuses the existing 8x8 multiplier blocks to compute the result over multiple clock cycles.

This document will analyze the trade-offs between two designs, focusing on key metrics such as power, hardware area, and control complexity, to determine the most effective architecture for the system.

Architecture Options

Both designs solve the same underlying math problem by breaking an int x uint multiplication into smaller stages. They reuse the four existing 8x8 multipliers to compute a 40-bit partial product in each stage. The difference lies entirely in where the control logic lives.

- Option A: Internal Multi-Cycle Unit (FSM Approach)

This architecture handles the multi-cycle execution entirely inside the PE. It acts like a black box to the rest of the system.

- Trigger Mechanism: The external controller issues a single SCALE instruction.

- Internal State: Once triggered, the PE's FSM immediately transitions from its IDLE state and takes autonomous control of the multiplier datapath.

- Execution: An internal 2-bit FSM sequences the operation over the next clock cycles. A 24-bit holding register is utilized to hold acc[31:8] through the sequence.

- Accumulation: In each cycle, the module computes the 40-bit partial product, applies the correct bit-shift, and adds it to the 64-bit accumulator.

- Completion: On the final cycle, the PE's FSM transitions back to its IDLE state and asserts the pe_ready signal, signaling the host that it can insert the next instruction.

- Option B: RISC-like Staged Instructions (Opcode Approach)

This architecture keeps the PE as simple as possible. It strips out the internal state machine and pushes all the scheduling responsibility onto the external controller.

- Trigger Mechanism: There is no single trigger. The external controller must issue a sequence of four distinct instructions: SCALE_0, SCALE_1, SCALE_2, and SCALE_3.

- Internal State: The PE holds no FSM and has no busy or done flags. It simply executes whatever opcode it receives on the current clock edge.

- Execution: Each opcode controls the PE directly. For example, SCALE_1 tells the PE to broadcast the second byte of op_A to the multipliers.

- Accumulation: Each opcode also dictates the exact bit-shift required before adding the 40-bit partial product to the accumulator.

- Completion: The operation finishes when the controller issues the final SCALE_3 instruction.

PPA and System-Level Complexity

Internal Multi-Cycle Unit (FSM Approach)

- Area: This design carries a small area penalty inside the PE. It requires instantiating dedicated sequential logic, specifically the flip-flops for the FSM, and the associated combinational logic to calculate the next state and drive the internal multiplexers.

- Performance (Latency & Timing): The latency for the multiplication phase is fixed. Timing-wise, generating control signals locally means the multiplexer select lines must pass through the FSM's state-decoding logic before data can enter the multipliers. However, because this FSM is only a 4-state, 2-bit counter, the decode logic is extremely shallow (typically a single gate delay). This delay is overweighed by the heavy propagation delay of the arithmetic blocks that follow it. Therefore, the FSM does not bottleneck the critical path.

- Power: The FSM consumes dynamic power as its state registers toggle every cycle during the operation. However, this is negligible regarding the other blocks of the PE.

- System Complexity: The system controller treats the PE as a fire-and-forget module. It issues a single SCALE3 command, and the PE handles the rest. The external controller only needs basic logic to pause issuing new instructions until the pe_ready signal is asserted.

RISC-like Staged Instructions (Opcode Approach)

- Area: By removing the FSM, the module strips out the flip-flops and associated combinational logic. The incoming instruction bus acts directly as the control lines, resulting in a negligible area improvement.

- Performance (Latency & Timing): The latency is identical. Because the control signals arrive directly from the opcode rather than being computed locally, the PE's internal control path is shorter. This shorter path can marginally improve the critical path delay.

- Power: Stripping the FSM negligibly improves the dynamic power consumption of the PE.

- System Complexity: The complexity is entirely offloaded from the hardware to the software or system controller.

Our Choice

To summarize the analysis, here is a high-level breakdown of the Power, Performance, Area, and System Complexity trade-offs:

| Metric | Internal FSM Approach (Selected) | Opcode Approach |
| --- | --- | --- |
| Power | Negligibly higher for the PE. | Negligibly higher for the system (external buses and controller are activated for each opcode). |
| Performance | Fixed latency. Internal changes do not require external controller modifications. | Fixed latency. Marginally shorter control path. |
| Area | Negligibly larger. | Negligibly smaller. |
| System Complexity | Low. Fire-and-forget: the external controller issues one command and waits for the pe_ready signal. | Low. The external controller issues the four opcodes in the correct sequence for the scaling to work. |

Table 1. Trade-offs summary.

The selected FSM approach trades a negligible amount of area and power for gains in reduced external controller complexity and cleaner software-hardware separation.