I/O Block Microarchitecture

High-Level Overview

The I/O Block is the data interface between the PE's external ports and its internal processing modules. It contains the two 32-bit operand input registers, the byte-select multiplexer, the output multiplexer, and the 8-bit output register. It registers incoming data, routes operand sub-fields to the appropriate consumers, selects the output source, and drives the external data output.

Interface

| Port Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 bit | Main system clock. |
| rst | Input | 1 bit | Active-low asynchronous reset. Clears all registers to zero. |
| operand_A | Input | 32-bit bus | External operand A data input. |
| operand_B | Input | 32-bit bus | External operand B data input. |
| valid_A | Input | 1 bit | External write-valid for operand A. |
| valid_B | Input | 1 bit | External write-valid for operand B. |
| chp_slct | Input | 1 bit | External chip select. Qualifies both operand captures. |
| pe_ready | Input | 1 bit | Ready signal from the Controller. Qualifies both operand captures. |
| accumulator | Input | 64-bit bus | Accumulator bus from the MAC & SCALE. Feeds the byte-select multiplexer. |
| activation_out | Input | 8-bit bus | Activated result from the PPU. Feeds the output multiplexer. |
| config_out | Input | 4-bit bus | Configuration register value from the Controller. Feeds the output multiplexer (zero-padded to 8 bits). |
| ppu_en | Input | 1 bit | From the Controller. Qualifies the output register write. |
| read_acc_en | Input | 1 bit | From the Controller. Selects byte_mux_out onto the output mux (overriding the activation_out default), enables the byte-select mux's accumulator input, and qualifies the output register write. |
| read_cfg_en | Input | 1 bit | From the Controller. Selects {4'b0, config_out} onto the output mux (overriding the activation_out default) and qualifies the output register write. |
| operand_A_reg | Output | 32-bit bus | Registered operand A. Drives MAC & SCALE port_A. Sub-fields are tapped by external wiring: [5:0] to PPU shamt_data, [3:0] to Controller cfg_data_in. |
| operand_B_reg | Output | 32-bit bus | Registered operand B. Drives MAC & SCALE port_B. |
| data_out | Output | 8-bit bus | Registered output data. Drives the external data_out bus. |

Table 1. I/O Block interface.

Input Register Bank

The I/O Block contains two 32-bit data registers.

operand_A_reg captures operand_A on the rising clock edge when valid_A, pe_ready, and chp_slct are all high. Otherwise it holds. Asynchronous reset clears it to zero.

operand_B_reg captures operand_B on the rising clock edge when valid_B, pe_ready, and chp_slct are all high. Otherwise it holds. Asynchronous reset clears it to zero.

operand_A_reg is driven out as a full 32-bit bus. The top level taps sub-fields as direct wire connections: bits [5:0] feed the PPU's shift-amount load data, and bits [3:0] feed the Controller's configuration load data. Bits [2:0] are consumed internally by the byte-select multiplexer.

operand_B_reg is driven out as a full 32-bit bus.

Byte-Select Path

The Byte-Select Path contains an 8:1 byte-select multiplexer. Its 64-bit input is the accumulator bus from the MAC & SCALE, treated as eight bytes; its select is operand_A_reg[2:0]; its output, byte_mux_out, is an 8-bit bus that feeds the Output Path.

Encoding of the select: 0 → accumulator[7:0], 1 → accumulator[15:8], …, 7 → accumulator[63:56].

The multiplexer's accumulator input is isolation-gated with read_acc_en. When read_acc_en is low, the gated input is forced to zero and byte_mux_out is held at zero regardless of select activity; mux switching is confined to READ_ACC_BYTE cycles.

Output Path

The Output Path contains a 3:1 output multiplexer and the 8-bit output register.

The output multiplexer combines three 8-bit sources into an 8-bit bus that feeds the output register. The three sources are:

| Source | Active During |
| --- | --- |
| Byte-select mux output (byte_mux_out) | read_acc_en |
| Zero-padded config register ({4'b0, config_out}) | read_cfg_en |
| Activation Block output (activation_out) | default (neither read enable asserted) |

Table 2. Output mux sources.

The output multiplexer defaults to activation_out. Two read overrides preempt the default: read_acc_en selects byte_mux_out, and read_cfg_en selects {4'b0, config_out}. The two read enables are mutually exclusive by Controller construction.

The output register's write enable, output_wr_en, is generated from ppu_en, read_acc_en, and read_cfg_en. The output register captures the output multiplexer result on the rising clock edge when output_wr_en is high.

When output_wr_en is low, the register holds its current value. The registered value drives the data_out output port.

In INT4 mode, data_out[3:0] holds the result and data_out[7:4] is don't-care.

In INT8 mode, the full data_out[7:0] bus holds the result.

Reset Behavior

When rst is driven low, all three registers are asynchronously cleared. operand_A_reg and operand_B_reg clear to 32'b0. The output register clears to 8'b0.