Post-Processing Unit (PPU) Microarchitecture

High-Level Overview

The Post-Processing Unit (PPU) is the final computational stage of the PE datapath. It receives the 64-bit signed accumulator from the MAC & SCALE module and produces an 8-bit quantized, activated result that is forwarded to the IO Block.

The PPU is organized as a two-stage cascade:

- Requantizer — scales the 64-bit accumulator down to an 8-bit signed value via arithmetic right-shift with rounding to the nearest integer, followed by a saturating clamp to the signed range of the active precision mode.

- Activation Block — applies the activation function selected by act_fn_sel to the Requantizer's 8-bit output, producing the PPU's final 8-bit result.

The two stages are connected in series; the accumulator-to-output datapath is purely combinational. The only sequential element in the PPU is the 6-bit shift-amount register, which resides inside the Requantizer. It is loaded during a SCALE instruction and held until the next SCALE.

The PPU participates in two PE instructions:

- SCALE (cycle 0): shamt_wr_en is asserted by the Controller and the shift-amount register inside the Requantizer is loaded from shamt_data. ppu_en is low, so activation_out is held at zero.

- EXEC_PP: ppu_en is asserted by the Controller for one cycle. The accumulator propagates combinationally through the Requantizer (using the previously stored shift amount) and the Activation Block; activation_out is valid in the same cycle and is sampled by the I/O Block output register on the following rising clock edge.

In every other cycle the PPU is idle: shamt_wr_en and ppu_en are both low, the shift-amount register holds its value, and activation_out is held at zero.

Interface

| Port Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 bit | Main system clock. Drives the shift-amount register inside the Requantizer (the only sequential element in the PPU). |
| rst | Input | 1 bit | Active-low asynchronous reset. Clears the 6-bit shift-amount register inside the Requantizer to zero. |
| accumulator | Input | 64-bit bus | Signed accumulator value from the MAC & SCALE. |
| shamt_data | Input | 6-bit bus | Shift-amount load data, sourced from the I/O Block. |
| shamt_wr_en | Input | 1 bit | Shift-amount register write enable, sourced from the Controller. Asserted on SCALE execution cycle 0. |
| mode4x4 | Input | 1 bit | Precision mode from the Controller's configuration register. High for INT4, low for INT8. |
| act_fn_sel | Input | 3-bit bus | Activation function select from the Controller's configuration register. |
| ppu_en | Input | 1 bit | PPU enable from the Controller. Asserted for one cycle during EXEC_PP. |
| activation_out | Output | 8-bit bus | Final quantized and activated result, driven to the I/O Block. |

Table 1. PPU interface.

Block Diagram (is necessary?)

Figure 1. Block-level microarchitecture.

Functional Operation

The PPU's externally visible behavior is fully determined by rst, shamt_wr_en, and ppu_en. The four cases below are mutually exclusive and cover every cycle.

- Reset. rst = 0 — the shift-amount register inside the Requantizer is asynchronously cleared to zero. activation_out is held at zero (ppu_en is low at reset).

- SCALE cycle 0. shamt_wr_en = 1, ppu_en = 0 — shamt_data is captured into the shift-amount register on the rising edge of clk. activation_out = 0.

- EXEC_PP. ppu_en = 1, shamt_wr_en = 0 — the accumulator propagates combinationally through the Requantizer (using the shift amount loaded by the previous SCALE) and the Activation Block, yielding a valid activation_out in the same cycle.

- Idle. shamt_wr_en = 0, ppu_en = 0 — the shift-amount register holds its value and activation_out = 0.

Requantizer

The Requantizer is the first stage of the PPU. It performs:

- Arithmetic right-shift of the 64-bit signed accumulator by the value held in the 6-bit shift-amount register, with rounding to the nearest integer.

- Saturating clamp of the rounded result to the signed range of the active precision mode, selected by mode4x4 (INT8 or INT4).

The 8-bit signed result is driven to the Activation Block as activation_in.

The Requantizer contains the 6-bit shift-amount register — the only sequential element in the PPU. The register is clocked by clk, write-enabled by shamt_wr_en (loaded from shamt_data), and asynchronously cleared to zero by rst. All other Requantizer logic is purely combinational.

Interface within the PPU

| Port | Connected To |
| --- | --- |
| accumulator[63:0] | PPU accumulator port |
| mode4x4 | PPU mode4x4 port |
| shamt_data[5:0] | PPU shamt_data port |
| shamt_wr_en | PPU shamt_wr_en port |
| clk | PPU clk port |
| rst | PPU rst port |
| activation_in[7:0] | Activation Block activation_in port |

Table 2. Requantizer interface within the PPU.

Activation Block

The Activation Block is the second and final stage of the PPU. It applies the activation function selected by act_fn_sel to the Requantizer's 8-bit signed output, with the input/output range determined by mode4x4. The supported functions and their encodings are defined in the PE Definition document.

The Activation Block is purely combinational. Its 8-bit output is qualified by ppu_en: when ppu_en = 0, activation_out is held at zero. When ppu_en = 1, activation_out carries the value computed by the selected activation function on activation_in.

Interface within the PPU

| Port | Connected To |
| --- | --- |
| activation_in[7:0] | Requantizer activation_in port |
| mode4x4 | PPU mode4x4 port |
| act_fn_sel[2:0] | PPU act_fn_sel port |
| ppu_en | PPU ppu_en port |
| activation_out[7:0] | PPU activation_out port |

Table 3. Activation Block interface within the PPU.

Reset Behavior

rst is an active-low asynchronous reset. It clears the 6-bit shift-amount register inside the Requantizer to zero; no other sequential state exists in the PPU. While rst = 0:

- The Requantizer's combinational output is the saturated form of the current accumulator with a zero shift amount.

- The Activation Block computes its selected function on that input, but because ppu_en is low at reset, activation_out is held at zero.