Requantizer Unit Testing Documentation

Overview

The Requantizer is a purely combinational module that takes a 64-bit signed accumulator value, applies an arithmetic right-shift with rounding to nearest integer, and clamps the result to a signed 8-bit or 4-bit range depending on the precision mode. The verification strategy uses directed testing to validate every architectural path and boundary condition, with expected values derived from a golden reference model.

Golden Reference Model

The testbench contains two SystemVerilog functions, golden_int8 and golden_int4, that compute the mathematically correct output for any input combination. Both functions perform the same rounding formula using 128-bit signed intermediate arithmetic to avoid overflow on extreme shift values:

rounded=x,  &shamt=0x+2shamt-1>>>shamt,  &shamt>0

The INT8 function then clamps the result to [−128, +127] and returns the lower 8 bits. The INT4 function clamps to [−8, +7] and sign-extends the 4-bit result to 8 bits. The 128-bit width is necessary because the rounding offset 2^(shamt−1) can reach 2^62, which when added to a max-positive 64-bit value would overflow a 64-bit signed variable.

Requantizer Reference Model

R=x,  &s=0x+22-12s,  &s>0

| Mode | Condition | Output |
| --- | --- | --- |
| INT8 (mode4x4 = 0) | R>127 | 127 |
| INT8 (mode4x4 = 0) | R<-128 | -128 |
| INT8 (mode4x4 = 0) | -128≤R≤127 | R7:0 |
| INT4 (mode4x4 = 1) | R>7 | sign_extend(7) |
| INT4 (mode4x4 = 1) | R<-8 | sign_extend(-8) |
| INT4 (mode4x4 = 1) | -8≤R≤7 | sign_extendR3:0 |

where sign_extend pads the 4-bit result to 8 bits: {{4{R[3]}}, R[3:0]}.

INT8 Mode Tests

- Zero-Shift Bypass

Test objective: Verify the datapath when shamt = 0, meaning no shifting or rounding takes place. The accumulator value should pass directly through saturation logic.

Test procedure: We applied four values with shamt = 0: an in-range value (+50), a large positive overflow (+50000), a large negative overflow (−50000), and zero.

Expected outcome: +50 passes through unchanged. +50000 clamps to +127. −50000 clamps to −128. Zero produces zero. This confirms the saturation logic works correctly when the barrel shifter is bypassed.

- Saturation Cliff

Test objective: Test the exact boundaries where saturation engages and disengages.

Test procedure: We constructed accumulator and shamt pairs that produce results landing exactly on or one step beyond the 8-bit boundaries: +127 (in range), +128 (one above), −128 (in range), −129 (one below). We also tested the rounding overflow case: +255 with shamt = 1 produces 127.5, which rounds to 128 and must clamp to +127.

Expected outcome: +127 and −128 pass through. +128 and −129 are clamped. The rounding overflow case confirms the sign-toggle detection catches a value that fits before rounding but overflows after.

- Precision Rounding

Test objective: Verify the rounding-to-nearest-integer behavior for various fractional results.

Test procedure: We applied values that produce known fractional results after division: +1.5 (3 >> 1), −1.5 (−3 >> 1), +1.25 (5 >> 2), +2.75 (11 >> 2), and −2.75 (−11 >> 2).

Expected outcome: +1.5 rounds to +2, −1.5 rounds to −1 (toward positive infinity), +1.25 truncates to +1, +2.75 rounds to +3, −2.75 rounds to −3. This confirms the round bit extraction and incrementer produce correct results for both positive and negative inputs with varying fractional parts.

- Fractional Zero Traps

Test objective: Verify rounding behavior at the zero boundary where the sign of the result depends on correct rounding direction.

Test procedure: We applied +1 with shamt = 1 (result = +0.5) and −1 with shamt = 1 (result = −0.5).

Expected outcome: +0.5 rounds to +1. −0.5 rounds to 0 (toward positive infinity, not toward negative infinity). This confirms the asymmetric behavior of round-half-up for signed values near zero.

- Absolute 64-bit Limits

Test objective: Stress the barrel shifter at maximum width by applying the largest possible 64-bit values with the maximum shift amount.

Test procedure: We applied 64'h7FFF_FFFF_FFFF_FFFF (max positive) and 64'h8000_0000_0000_0000 (max negative) with shamt = 63. Expected values are computed by the golden reference model.

Expected outcome: Both cases produce small in-range results after the massive shift. This confirms the two-stage barrel shifter operates correctly across its full 65-bit width with all shift select bits active.

- Complete Underflow

Test objective: Verify that small values shifted by large amounts flush cleanly to zero.

Test procedure: We applied +100 and −100 with shamt = 20. Both values are far too small to survive a division by 2^20.

Expected outcome: Both produce zero. This confirms the shifter and rounding logic don't leave residual bits or sign-extension artifacts when the input is entirely shifted away.

- Large Values with Moderate Shift

Test objective: Test the interaction of large accumulator values with moderate shift amounts that produce in-range results.

Test procedure: We applied +1000000 and −1000000 with shamt = 20. The expected results are computed by the golden model.

Expected outcome: The golden model determines the correct rounded value. This tests a realistic scenario where the accumulator holds a large partial sum that gets scaled down to an 8-bit range.

- Power-of-Two Division

Test objective: Test exact and overflow-producing divisions by powers of two.

Test procedure: We applied 1024 with three shift amounts: shamt = 10 (1024/1024 = 1, exact), shamt = 5 (1024/32 = 32, in range), and shamt = 3 (1024/8 = 128, one above the positive boundary).

Expected outcome: shamt = 10 produces exactly 1. shamt = 5 produces 32. shamt = 3 produces 128 which clamps to +127. This confirms correct behavior for clean power-of-two inputs where no rounding is involved, isolating the saturation logic from rounding effects.

INT4 Mode Tests

- Zero-Shift Bypass INT4

Test objective: Verify the INT4 saturation path when shamt = 0.

Test procedure: We applied four values with mode4x4 = 1 and shamt = 0: +5 (in range), −6 (in range), +50 (overflow), and −50 (overflow). Expected values are computed by the golden INT4 model.

Expected outcome: +5 and −6 pass through with sign extension. +50 clamps to +7. −50 clamps to −8. This confirms the cascaded 61-bit overflow trees and INT4 clamping mux operate correctly when the shifter is bypassed.

- INT4 Saturation Cliff

Test objective: Test the exact INT4 boundaries.

Test procedure: We constructed pairs producing results at +7 (in range), +8 (one above), −8 (in range), and −9 (one below).

Expected outcome: +7 and −8 pass through. +8 and −9 are clamped. This confirms the 4-bit truncation overflow detection engages at the correct boundary.

- INT4 Rounding Overflow

Test objective: Test the INT4 rounding overflow path specifically.

Test procedure: We applied +15 with shamt = 1. The result is 7.5, which rounds to 8, exceeding the INT4 maximum of +7.

Expected outcome: The output clamps to +7. This confirms the sign-toggle detection on bit [3] catches INT4 rounding overflow.

- INT4 Precision Rounding

Test objective: Verify rounding behavior in INT4 mode.

Test procedure: We applied +3 with shamt = 1 (+1.5) and −3 with shamt = 1 (−1.5).

Expected outcome: +1.5 rounds to +2. −1.5 rounds to −1. The shared 8-bit incrementer produces correct 4-bit results in its lower bits.

- INT4 Fractional Zero Traps

Test objective: Verify INT4 rounding at the zero boundary.

Test procedure: We applied +1 and −1 with shamt = 1.

Expected outcome: +0.5 rounds to +1. −0.5 rounds to 0. Same rounding behavior as INT8, confirming the shared incrementer and round bit extraction are mode-independent.

- INT4 Sign Extension

Test objective: Verify that INT4 results are correctly sign-extended to 8 bits on the output.

Test procedure: We applied −4 and +3 with shamt = 0 in INT4 mode. Both fit within the 4-bit range.

Expected outcome: −4 produces 8'hFC (1111_1100) and +3 produces 8'h03 (0000_0011). This confirms the output mux correctly replicates bit [3] into the upper four bits.

- INT4 Absolute 64-bit Limits

Test objective: Stress the barrel shifter at full width in INT4 mode.

Test procedure: We applied the maximum positive and maximum negative 64-bit values with shamt = 63. Expected values are computed by the golden INT4 model.

Expected outcome: Both cases produce small values that fit within the INT4 range. This confirms the cascaded overflow trees correctly evaluate the 61-bit check at maximum shift depth.

- INT4 Complete Underflow

Test objective: Verify clean underflow to zero in INT4 mode.

Test procedure: We applied +3 with shamt = 20.

Expected outcome: The result is zero. The small input is entirely shifted away, confirming no residual bits survive in INT4 mode.

Mode Switching Tests

- Same Input, Different Modes

Test objective: Verify that the mode4x4 signal correctly selects between the INT8 and INT4 clamping paths for the same input.

Test procedure: We applied +100 with shamt = 0 and toggled mode4x4 between 0 and 1 without changing the input.

Expected outcome: In INT8 mode, +100 is in range and passes through. In INT4 mode, +100 exceeds +7 and clamps. This confirms the output mux responds correctly to mode changes.

- Fits INT8 But Not INT4

Test objective: Verify correct behavior for a value that is valid in INT8 but overflows in INT4.

Test procedure: We applied +10 with shamt = 0 and toggled mode4x4.

Expected outcome: INT8 mode outputs +10. INT4 mode clamps to +7. This confirms the two saturation paths are independent and the mode select correctly routes between them.