Activation Block Unit Testing Documentation

Overview

The Activation Block is a purely combinational hardware module designed for a neural network accelerator. It applies mathematical activation functions to an incoming data stream. It contains dual-precision parallel datapaths (INT8 and INT4), linear function logic blocks, non-linear lookup tables (LUTs), power-saving input gating, and output selection multiplexers. The verification strategy uses directed testing organized into four groups to cover every mathematical boundary condition and architectural path, followed by a constrained random phase that validates structural invariants over 10,000 cycles. Expected values are derived from a golden reference model that combinationally evaluates against the DUT on every input transition.

Golden Reference Model

The testbench contains a combinational golden reference model implemented as a SystemVerilog function (golden_model). The model maintains a software-equivalent implementation of the linear mathematical functions and exact data mappings for the non-linear LUTs.

Combinational Evaluation

Since the DUT has no sequential elements, the model evaluates all inputs (activation_in, mode4x4, act_fn_sel) immediately upon stimulation. To prevent simulation race conditions, the comparison is delayed by 1 ns to allow the combinational logic inside the DUT to fully settle. The model mimics the hardware's multiplexing behavior: it explicitly computes both 8-bit and 4-bit results independently based on the act_fn_sel input, and then formats the final expected output based on mode4x4 (including necessary 4-bit zero-padding). Violating the padding expectation—for example, allowing sign extension to bleed into the upper 4 bits during INT4 mode—will cause instant failures in the model comparison.

Stimulus Tasks

The testbench provides a unified checker task built around the model evaluator: check_output(test_name, exp_val) drives the combinational inputs, waits 1 ns for signal propagation, and evaluates the DUT's activation_out. If exp_val is provided, it performs a strict equality check (!==) against the hardcoded expected value. If exp_val is omitted, it dynamically queries golden_model for the expected vector and performs the comparison.

Test Plan

Group 1: Directed Testing – Linear Functions

This group explicitly tests the mathematical logic of the Identity, ReLU, and Leaky ReLU paths.

Test 1.1: Zero Crossing (INT8 ReLU). Test objective: Verify that ReLU correctly passes positive numbers and clamps negative numbers to zero. Test procedure: Drive mode4x4 = 0, act_fn_sel = 3'b001, and sweep activation_in across 1, 0, and -1. Expected outcome: Outputs are 8'd1, 8'd0, and 8'd0 respectively.

Test 1.2: Positive and Negative Extrema (INT8). Test objective: Verify no overflow/underflow anomalies occur in Identity, and verify LReLU correctly shifts the maximum negative value. Test procedure: Drive activation_in = 8'h7F with Identity (3'b000), and 8'h80 with LReLU2 (3'b011). Expected outcome: Identity outputs 8'h7F. LReLU2 drops the correct bits via arithmetic shift while maintaining sign extension (8'h80 >>> 2 = 8'hE0 / -32).

Test 1.3: Positive and Negative Extrema (INT4). Test objective: Verify the INT4 sign bit (bit 3) is correctly interpreted by the ReLU conditional. Test procedure: Drive mode4x4 = 1, act_fn_sel = 3'b001. Drive activation_in = 8'h07 and 8'hF8. Expected outcome: Output is padded 8'h07 for the positive input, and clamped to 8'h00 for the negative input.

Group 2: Directed Testing – Non-Linear Functions (LUTs)

Because these are manually encoded tables, they are checked against hard-coded known-good bounds to prevent copy-paste errors.

Test 2.1: LUT Boundary Conditions (INT8). Test objective: Verify the top, bottom, and middle mappings of the 256-entry INT8 LUTs. Test procedure: Drive mode4x4 = 0, act_fn_sel = 3'b100 (Tanh). Assert activation_in values of 8'h80, 8'h00, and 8'h7F. Expected outcome: Tanh outputs exactly match hardcoded golden bounds (8'b10000000, 8'b00000000, 8'b01111111).

Test 2.2: LUT Boundary Conditions (INT4). Test objective: Verify the extrema mappings of the 16-entry 4-bit LUTs. Test procedure: Drive mode4x4 = 1, act_fn_sel = 3'b101 (Sigmoid). Assert activation_in values of 8'h08 (max negative) and 8'h07 (max positive). Expected outcome: Sigmoid outputs exactly match hardcoded golden bounds (8'b0000_0000 and 8'b0000_0111).

Group 3: Datapath & Control Logic

This group ensures the datapath muxing, power gating, and padding behave correctly outside of mathematical operations.

Test 3.1: INT4 Output Padding. Test objective: Verify that INT4 mode correctly pads the upper 4 bits with zeros and prevents sign extension leakage. Test procedure: Drive mode4x4 = 1, act_fn_sel = 3'b000 (Identity), and activation_in = 8'hFF. Expected outcome: Output is exactly 8'h0F. The upper 4 bits are strictly zeroed out.

Test 3.2: Invalid Function Selection. Test objective: Verify the default case in the output selection multiplexer defaults safely to zero. Test procedure: Drive act_fn_sel with an out-of-bounds state (e.g., if the simulator permits injecting 3'bX or logic values outside standard execution). Expected outcome: activation_out evaluates to 8'h00.

Group 4: Constrained Random Verification

Test 4.1: 10,000-cycle Combinational Blender. Test objective: Exercise arbitrary, simultaneous toggling of datapath inputs, precision modes, and function selectors to expose combinational glitches, overlapping assignments, or unmapped case statements. Test procedure: Over a loop of 10,000 iterations, the testbench generates a completely random activation_in, a random act_fn_sel, and a random mode4x4 state. The golden reference model evaluates the randomized inputs in lockstep with the DUT. Additionally, structural invariants are checked on every cycle: Outputs generated while mode4x4 = 1 must never contain high bits in activation_out[7:4]. Expected outcome: All 10,000 output comparisons pass on every cycle. No structural invariant violations occur. This validates the Activation Block's resilience to arbitrary input sequences without generating latches or combinational loops.