Activation Block Architectural Trade-Offs

Introduction

The objective of this document is to define architectures and explore the trade-offs between them for the Activation Block. This block requires a combination of memory-based Look-Up Table (LUT) implementation and synthesizable logic to support non-linear functions. We have defined four potential architectural topologies for evaluation.

Architecture Options

This section details the architectural topologies evaluated for the Activation Block. In all configurations, the block is designed to support 8 distinct activation functions. Four non-linear functions (Tanh, Sigmoid, GELU, SiLU) require lookup tables, while the remaining four (ReLU, Identity, and two Leaky ReLU variants with different alpha coefficients) are implemented via a parallel synthesizable logic path in every configuration. 	To minimize dynamic power consumption, every configuration incorporates an enable mechanism to prevent unnecessary toggling and power waste.

- Option A: SRAM (Flexible RAM)

This option utilizes a single SRAM block (256 rows × 8 bits). The RAM serves as a fully programmable LUT. Unlike the ROM options, the content is not fixed at fabrication; the user can load any arbitrary activation function into the memory, allowing for post-silicon updates or support for custom functions.

Trade-off: This architecture offers maximum flexibility. However, because it relies on a standard 6-transistor bitcell, it consumes significantly more area per bit than the ROM alternatives. It also suffers from high static power per bit due to 6T bitcell leakage across all 256 rows, and requires high dynamic power to precharge the differential bitlines during reads.

- Option B: ROM – Unified Memory

This option utilizes a single, monolithic ROM block (1024 rows × 8 bits) to store the four non-linear functions: Tanh, Sigmoid, GELU, and SiLU. The specific function is accessed by concatenating the activation selection bits with the input data to form the address in the unified block.

Trade-off: This setup avoids the complex output routing and multiplexing required by the banked architecture. However, it requires a complex 10-to-1024 decoder, which creates a massive area overhead. Furthermore, the long 1024-cell bitlines create high parasitic capacitance, making this option slower and more power-hungry during active reads compared to Option C.

- Option C: ROM – Banked Memory

This approach physically divides the memory into four independent banks (256 rows × 8 bits each), one per non-linear function. Each bank has its own local decoder, and the activation selection signal enables only the relevant bank's decoder. All other banks remain fully idle; no wordline or bitline activity occurs in inactive banks.

Trade-off: Full bank isolation minimizes dynamic power, since only one decoder and one set of bitlines are active at any time. The cost is a larger physical footprint due to the four duplicated decoders and additional routing between banks and the output multiplexer.

- Option D: Logic-Synthesized LUT (Combinational Implementation)

This option implements each non-linear function as a combinational case statement in RTL, which is then mapped into synthesizable standard-cell logic. There are no bitcells, decoders, sense amplifiers, precharge circuits, or bitlines.

Trade-off: This achieves the smallest area and lowest power, but the values are fixed at synthesis time and cannot be reprogrammed post-silicon. This approach only remains efficient for small table sizes; for hundreds or thousands of entries, a memory-based structure would be more area-efficient.

| Metric | Option A: SRAM | Option B: ROM (Unified) | Option C: Split ROM | Option D: Synthesized Logic (Selected) |
| --- | --- | --- | --- | --- |
| Configuration | Single Block (256 × 8 bits) | Single Block (1024 × 8 bits) | 4 Banks (4 × [256 × 8 bits]) Independent Decoders | Combinational standard cells (no memory structure) |
| Area | 1523.86 μm² | 1796.347 μm² | 3744.90 µm² | 320.42 μm² |
| Critical Path | 0.5952 ns | 0.843 ns | 0.665 ns | 0.74 ns |
| Dynamic Power | 491.59 μW | 404.46 μW | 326.70 µW | 15.6 μW |
| Static Power | 2.02 μW | 3.95 μW | 10.09 µW | 63 nW |

PPA Table

Table 1. Post-synthesis PPA comparison.

Note: The Option D area figure represents the sum of all four individually synthesized non-linear functions (Tanh, Sigmoid, GELU, SiLU), serving as an upper bound. ReLU, Identity, and Leaky ReLU variants are implemented via synthesizable logic in all configurations and are excluded from this comparison. The power and critical path figures for Option D represent the worst case across the four non-linear functions.

Our Choice

We selected the logic-synthesized LUT (Option D). The synthesis tool implements each 256-entry table as combinational logic, eliminating all memory peripheral overhead at a fraction of the area and power cost of a memory-based structure. ReLU, Identity, and Leaky ReLU variants remain implemented via synthesizable logic, as in Options B and C.

Appendix: Activation Functions

| Activation Function | Logical Role & Behavior | Critical Application | Risk of Exclusion |
| --- | --- | --- | --- |
| ReLU (Rectified Linear Unit) | Pass-through / Zeroing | Computer Vision: The standard for image recognition (e.g., smart security sensors). Computationally efficient for deep networks. | PE would be inefficient and slow for standard image tasks. |
| Identity | Pass-through | Residual Learning: Crucial for "Skip Connections" in deep networks (e.g., ResNet) to prevent vanishing gradients, and for Linear Output Layers in regression tasks. | The PE would fail to support modern deep architectures (ResNets) or regression models, severely limiting its flexibility and accuracy on complex tasks. |
| Leaky ReLU (α = 0.25) | Pass-through with scaled negative leakage | General-purpose training: Prevents "dying ReLU" by allowing a small gradient for negative inputs, improving convergence in networks sensitive to dead neurons. | PE would lack a standard mitigation for the dying-neuron problem, limiting robustness on deeper networks. |
| Leaky ReLU (α = 0.015625) | Pass-through with attenuated negative leakage | Low-noise inference: A near-ReLU response that preserves a minimal negative signal, commonly used in lightweight edge classifiers where aggressive negative suppression is preferred. | PE would offer no intermediate option between full ReLU zeroing and the larger α variant, reducing flexibility. |
| Tanh (Hyperbolic Tangent) | Zero-Centered Mapping | Voice Processing: Essential for the "Cell State" in recurrent networks used in voice activation (e.g., smart toys). | PE would struggle to maintain long-term memory, failing at sequence tasks. |
| Sigmoid | Probability Switch | Required for "Forget/Update Gates". Decides what information to keep. | PE could not run standard LSTMs or GRUs without Sigmoid, breaking voice/audio applications. |
| GELU (Gaussian Error Linear Unit) | Probabilistic Smoothing | Modern AI (Transformers) | PE would be stuck supporting only older (pre-2018) models |
| SiLU (Sigmoid Weighted Linear Unit) | Self-Gating, allows small negative values to propagate | The standard activation for MobileNetV3 and EfficientNet, which are specifically designed for low-power edge devices | PE could not natively accelerate the most popular "mobile-optimized" networks, forcing them to fallback to less accurate approximations. |

Table 2. Activation functions.