Architectural Trade-Offs: Multiplier Implementations for MAC Operations

Introduction

This document evaluates two different architectural approaches for implementing the multi-precision multiplier block within the Processing Element (PE). The module must handle MAC8 (8x8) operations as well as MAC4 (4x4) operations for neural network computations. Because the PE relies heavily on multiply-accumulate density, we need an architecture that balances silicon area, timing constraints, and maximum operation throughput. This document will analyze the trade-offs between two designs to determine the most effective architecture for our system.

Architecture Options

This multi-precision process can be implemented in one of two ways.

- Option A: Energy-Saving Implementation (ESI)

This method reuses the signed 8x8 multipliers for 4x4 multiplications. It relies on a standard multiplier structure, which allows it to accumulate four 4x4 multiplications at each clock cycle.

- Option B: High-Throughput Implementation (HTI)

This method reconfigures the 8x8 multipliers so they can function as two 4x4 multipliers as well. It does this by using four small 4x4 signed multipliers to implement the 8x8 signed multiplier. Because of this structural change, the design can accumulate eight 4x4 multiplications at each clock cycle.

PPA Evaluation

ESI

- Pros: This approach offers a shorter critical path for each multiplication process, and is also more area conservative.

- Cons: The main drawback is lower throughput, as it is limited to only four 4x4 operations per cycle.

HTI

- Pros: The primary advantage is high throughput, allowing the hardware to perform eight 4x4 signed multiplications per cycle. By using this method, the 8x8 signed multiplier is inherently multi-precision. It reuses the four 4x4 multipliers to compute both the 8x8 and the two 4x4 multiplications simultaneously.

- Cons: This method creates a longer critical path for each multiplication. It is more area consuming because we can no longer optimize the 8x8 multiplications as a single block.

Our Choice

After evaluating the options, we determined that the high-throughput multiplier was the most suitable choice for our PE design.

Here are the post-synthesis PPA results for both architectures:

| Metric | Energy-Saving Approach | High-Throughput Approach (Selected) |
| --- | --- | --- |
| Throughput (per cycle) | 4 MAC4 operations | 8 MAC4 operations |
| Power (per cycle) | ~32.3 µW | ~45.5 µW |
| Power Efficiency (per MAC4-op/cycle) | ~8.08 µW | ~5.69 µW |
| Area | ~119.07 µm² | ~214.33 µm² |
| Critical Path | ~1.07 nsec | ~1.19 nsec |

Our reasons for this implementation choice are:

- Superior Efficiency: The design delivers 2x throughput for MAC4 operations with less than a 1.5x increase in power consumption, resulting in a net improvement in energy efficiency.

- Negligible Area Impact: While the architecture requires approximately 2x the area of the energy-saving option, the absolute area penalty is minimal. Given that the PE contains only four multipliers, and each multiplier is very small in either option, this gain is negligible.

- Timing Preservation: The throughput gain is achieved with a negligible impact on the critical path, ensuring the target frequency is maintained.

Appendix: Workload-Weighted Power Efficiency Analysis

The initial PPA evaluation shows the High-Throughput Implementation (HTI) consuming ~45.5µW compared to the Energy-Saving Implementation (ESI) at ~32.3µW. However, these figures represent the worst-case dynamic power during continuous 8x8 operations. To evaluate the true energy efficiency of the multiplier within the PE, we must analyze the power consumption over time, weighted by the specific operating modes of the system.

Dynamic Power Reduction in 4x4 Mode

During MAC4 operations, the HTI does not draw its baseline ~45.5µW. The architecture implements operand isolation on the cross-term multipliers. When the 4x4 mode is active, the inputs to these sub-blocks are gated to zero (4'b0000). This prevents unnecessary toggling within half of the multiplier's logic gates, reducing dynamic power consumption during MAC4 mode (a ~10μW power decrease).

Time-Weighted Energy Comparison

We can evaluate both architectures based on the expected workload distribution of the system. A reasonable assumption is that the PE operates in a half-dormant MAC4 mode for 99% of its active time and enters full MAC8 mode for 1% of the time. The density of operations during the MAC8 state is 15x to 30x higher than during the dormant MAC4 state.

We can define the average power consumption (Pavg) proportionally based on the rate of 4x4 operations (R4), the energy per 8x8 operation (EMAC8), and the energy per 4x4 operation (EMAC4). Assuming an average workload multiplier of 20x for the active state (R8=20∙R4), the time-weighted average power simplifies to:

Pavg∝0.01∙R8∙EMAC8+0.99∙R4∙EMAC4

=R4∙0.2∙EMAC8+0.99∙EMAC4

This relationship demonstrates that the overall power consumption is heavily dominated by the energy efficiency of the MAC4 operations (0.99 weighting) rather than the MAC8 operations (0.2 weighting).

The HTI accumulates eight 4x4 multiplications per clock cycle, whereas the ESI only accumulates four. Because the HTI achieves 2x the throughput for MAC4 instructions, the energy required per 4x4 operation (EMAC4) is effectively halved compared to the ESI. When combined with the internal operand isolation, this massive reduction in EMAC4 heavily outweighs the slight increase in EMAC8 during the 1% active state.