ALU Verification and Unit Testing Documentation

Overview

The Arithmetic Logic Unit (ALU) acts as the primary arithmetic core of the processing system, computing various multiply-accumulate operations based on a 5-bit instruction. The verification strategy combines targeted directed testing to validate specific architectural routing, the multi-cycle MULT32 sequence, and physical hardware boundaries. Alongside this, Constrained Random Verification (CRV) utilizes a Reference Model to stress the hardware with randomized commands. This dual approach ensures the datapath and internal state machine maintain synchronization and mathematical accuracy across a massive, unpredictable execution space.

Reference Model Definition

State Definition: 64-bit signed two's complement Accumulator (Acc).

| Command Name | Command Code | Mathematical State Update |
| --- | --- | --- |
| NOP | 0 | Accn+1=Accn |
| RST_ACC | 1 | Accn+1=0 |
| MAC4 | 2 | Accn+1=Accn+i=07(signedOpA[4i+3:4i]×signedOpB[4i+3:4i] |
| MAC8 | 3 | Accn+1=Accn+i=04(signedOpA[8i+7:8i]×signedOpB[8i+7:8i] |
| ADD_BIAS | 4 | Accn+1=Accn+signedOpB |
| MULT32 | 5 | Accn+1=signedAccn31:0×unsignedOpB |

Directed Testing

Reset, Initialization, and State Control

- Async Hardware Reset

Test Objective: To evaluate the hardware's initialization and asynchronous reset behavior.

Test Procedure: We asserted the active-low rst pin to zero before the first positive clock edge.

Expected Outcome: This ensures the reset completely overrides the clock, clearing the internal FSM state, dropping status flags, flushing the 64-bit accumulator and the 32-bit multi-cycle holding register to a clean zero state.

- NOP Holds State

Test Objective: To confirm the idle state stability of the datapath.

Test Procedure: We issued the NOP command while feeding constantly changing data into the op_A and op_B input pins.

Expected Outcome: This proves the datapath properly ignores external inputs when commanded to sit idle, guaranteeing the 64-bit accumulator acts as a stable memory register and holds its current state without corruption.

- Synchronous RST_ACC

Test Objective: To verify the software-triggered accumulator clear command.

Test Procedure: We sent the RST_ACC instruction.

Expected Outcome: This confirms that the 64-bit accumulator flushes to zero synchronously on the next clock edge. This allows the system to clear the running sum between different vector calculations without triggering a global hardware reset.

- Async Reset Mid-Operation

Test Objective: To test the robustness of the asynchronous reset during active FSM sequences.

Test Procedure: We initiated a 4-cycle MULT32 command, waited until the second cycle, and pulled the rst pin low independent of the clock edge.

Expected Outcome: This proves the hardware instantly aborts the calculation, resets the FSM cycle counter, drops the busy flag, and clears both the main accumulator and the internal holding register.

Single-Cycle Datapath Execution

- ADD_BIAS Operation

Test Objective: To validate the single-cycle bypass routing path for the bias instruction.

Test Procedure: We issued the ADD_BIAS command with a known 32-bit op_B value while tying op_A to zero.

Expected Outcome: This confirms the 32-bit input bypasses the multiplier array. It verifies the data is properly sign-extended to 64 bits and added directly to the accumulator in exactly one clock cycle.

- MAC4 Operation

Test Objective: To verify the 4x4 parallel multiply-accumulate hardware configuration..

Test Procedure: We sent the MAC4 command with inputs explicitly formatted as eight distinct 4-bit numbers.

Expected Outcome: This ensures all eight separate partial products are correctly sign-extended to 64 bits and summed into the accumulator without any mathematical cross-talk or carry leakage.

- MAC8 Operation

Test Objective: To verify the 8x8 parallel multiply-accumulate hardware configuration.

Test Procedure: We sent the MAC8 command with specific 8-bit chunked values on both input ports.

Expected Outcome: This ensures all four underlying 16-bit intermediate products are correctly sign-extended and summed into the 64-bit accumulator in a single clock cycle.

- MAC4 with Negative Nibbles

Test Objective: To verify the sign-extension of negative 4-bit products into the 64-bit accumulator.

Test Procedure: We issued the MAC4 command with operands explicitly constructed to contain negative 4-bit signed values. Each byte of op_A contained the nibbles -1 and +1, while each byte of op_B contained the nibbles +2 and +3, producing a mix of positive and negative partial products across all eight lanes.

Expected Outcome: This ensures the datapath correctly interprets each nibble as a signed 4-bit two's complement value, properly sign-extends the resulting 8-bit products to 64 bits, and accumulates the mixed positive and negative partial sums without corrupting the sign bits or leaking carries between adjacent nibble lanes.

- ADD_BIAS with Negative Value (Isolated)

Test Objective: To validate the sign-extension bypass path for a negative 32-bit bias in complete isolation.

Test Procedure: We cleared the accumulator and issued the ADD_BIAS command with the maximum negative 32-bit value (0x80000000) while tying op_A to zero.

Expected Outcome: This confirms the 32-bit bias correctly sign-extends to a full 64-bit negative number when routed through the bypass path. It proves the upper 32 bits of the accumulator are filled with ones, matching the expected two's complement representation, and that no bits are dropped or zeroed during the extension.

- MAC8 with Negative Bytes from Zero

Test Objective: To isolate MAC8 sign-extension behavior without any prior accumulator state.

Test Procedure: We cleared the accumulator and issued a single MAC8 command with operands specifically chosen to mix extreme signed 8-bit values across all four byte lanes: maximum positive (+127), maximum negative (-128), small positive (+1), and near-zero negative (-1) values.

Expected Outcome: This ensures each of the four independent multiplier lanes handles its signed operands correctly and that the resulting 16-bit products are individually sign-extended to 64 bits before summation. It proves the accumulator correctly represents large negative results from a clean zero starting state.

Datapath Reconfiguration

- Mode Switching MAC8 then MAC4 Back-to-Back

Test Objective: To verify the multiplier array reconfigures cleanly between 8x8 and dual 4x4 modes on consecutive clock cycles.

Test Procedure: We cleared the accumulator and immediately issued a MAC8 command followed by a MAC4 command on the very next clock edge, without inserting any idle cycles in between.

Expected Outcome: This proves the combinational mode4x4 control signal on the multiplier array switches instantaneously between operating modes. It confirms the cross-term operand isolation logic fully disengages during the MAC4 cycle and that no residual routing state from the previous MAC8 operation corrupts the independent 4x4 partial products. The final accumulator value must equal the exact mathematical sum of both operations.

Multi-Cycle FSM and Control Handshaking

- Done Flag Pulse Width

Test Objective: To verify that the done output flag is a single-cycle pulse as specified by the microarchitecture.

Test Procedure: We triggered a MULT32 sequence and waited for the done flag to assert high. On the very next clock edge, we issued a NOP command and immediately sampled the done flag again.

Expected Outcome: This confirms the done flag drops back to zero exactly one clock cycle after it asserts. It proves the flag behaves as a clean single-cycle pulse and does not remain stuck high, ensuring downstream logic that monitors the done signal for completion events will not register false triggers on subsequent cycles.

- RST_ACC Preserves Status Flags

Test Objective: To verify that the RST_ACC instruction exclusively clears the accumulator without disturbing the done or busy status flags.

Test Procedure: We triggered a MULT32 sequence and waited for the done flag to pulse high. On the very next clock edge, we issued an RST_ACC command and sampled all three outputs: the accumulator, the done flag, and the busy flag. We then issued a NOP command on the following cycle and sampled the done flag again.

Expected Outcome: This confirms the accumulator flushes to zero while the done flag remains high through the RST_ACC cycle. In the RTL, RST_ACC sits at the top of the priority chain and skips the branch that clears the done flag. The subsequent NOP enters the normal command path, which correctly clears the done flag to zero. This proves the RST_ACC instruction is an accumulator-only clear that does not modify the FSM status flags, matching the microarchitectural specification.

- Busy Flag Asserted

Test Objective: To check the immediate FSM handshake initiation for multi-cycle commands.

Test Procedure: We sent the MULT32 command and sampled the busy output flag immediately after the active clock edge.

Expected Outcome: This ensures the FSM raises the busy flag on the very first cycle of a multi-cycle sequence to prevent the external controller from sending conflicting commands.

- MULT32 Sequence & Routing

Test Objective: To validate the 32x32 multi-cycle calculation and internal feedback loop.

Test Procedure: We triggered MULT32 with a known scaling factor on op_B, attempted to send a new command while the hardware was calculating, and waited for the done pulse.

Expected Outcome: This verifies the datapath successfully ignores the external op_A pin and routes the internal accumulator feedback instead. It also confirms the 4-cycle FSM math is correct, the done flag pulses properly, and the module successfully ignores new incoming commands while busy.

- Back-to-Back Stress

Test Objective: To verify continuous, back-to-back accumulation stability.

Test Procedure: We fired three consecutive MAC8 commands on successive clock edges without inserting any NOP or idle cycles.

Expected Outcome: This proves the datapath can register inputs, calculate sums, and accumulate new data every single clock cycle without dropping operations or creating structural hazards.

- FSM Immediate Re-trigger

Test Objective: To test the agility and state transitions of the multi-cycle FSM.

Test Procedure: We initiated a MULT32 sequence, waited for the done flag to pulse, and immediately issued a new MULT32 command on that exact same clock cycle.

Expected Outcome: This confirms the internal FSM can jump directly from its final calculation state back to the initial execution state, capturing the new operands and restarting the sequence without requiring a wasted idle cycle in between.

Mathematical Boundaries and Edge Cases

- Negative Math & Sign Extension

Test Objective: To validate two's complement arithmetic and sign-extension across different instruction paths.

Test Procedure: We loaded a negative bias using the ADD_BIAS instruction, followed by a MAC8 operation utilizing negative operands.

Expected Outcome: This ensures the datapath properly treats op_A and op_B as signed values during standard vector operations. It proves the hardware correctly sign-extends all intermediate products to 64 bits and performs accurate two's complement subtraction.

- Unsigned MULT32 Scaling

Test Objective: To verify the dynamic sign control logic during 32x32 scalar multiplication.

Test Procedure: We loaded a positive base value and multiplied it by the maximum 32-bit unsigned value (0xFFFFFFFF) using the MULT32 instruction.

Expected Outcome: This confirms the hardware dynamically forces op_B to act as a strictly unsigned scaling factor during a multi-cycle operation. It proves the logic prevents large magnitude numbers from being incorrectly interpreted as negative values.

- Max Positive Carry Ripple

Test Objective: To stress test the combinational limits of the intermediate 32x8 adder tree.

Test Procedure: We multiplied the maximum 32-bit positive number (0x7FFFFFFF) by the maximum 32-bit unsigned scaling factor (0xFFFFFFFF).

Expected Outcome: This forces the maximum possible number of carry bits to ripple through the 24-bit and 40-bit adders. It proves the combinational logic calculates massive positive sums accurately without truncation or carry dropping.

- Max Negative Asymmetry

Test Objective: To test the hardware boundaries against the two's complement asymmetric range.

Test Procedure: We multiplied the maximum 32-bit negative number (0x80000000) by the maximum 32-bit unsigned scaling factor (0xFFFFFFFF).

Expected Outcome: This verifies the adder tree and shift logic can handle the absolute largest negative intermediate products without overflowing or corrupting the critical sign bit.

- Zero-Crossing Wrap-Around

Test Objective: To validate that the accumulator can wrap around to 0.

Test Procedure: We loaded the 64-bit accumulator with a value of -1 (all 1s in 64-bit hexadecimal) and added exactly 1 to it.

Expected Outcome: This ensures the hardware correctly mimics standard two's complement arithmetic by rolling over cleanly to zero without freezing, throwing an error, or dropping into an unknown state.

- MULT32 with Nonzero Upper Accumulator Bits

Test Objective: To verify that the MULT32 instruction correctly captures only the lower 32 bits of the accumulator and that nonzero upper bits do not corrupt the calculation.

Test Procedure: We loaded a large positive base value and executed a MULT32 sequence that produced a 64-bit result with nonzero upper bits. We verified this intermediate result, then immediately triggered a second MULT32 sequence with a known scaling factor.

Expected Outcome: This confirms the internal feedback multiplexer routes strictly accumulator[31:0] to the multiplier array on the first cycle of a MULT32 sequence. It proves the 32-bit holding register captures only the lower half, completely discarding the upper 32 bits. The second multiplication result must match the mathematical product of the lower 32 bits treated as a signed value multiplied by the unsigned scaling factor, with no interference from the previous upper-half residue.

- 64-bit Accumulator Rollover

Test Objective: To force a genuine overflow across the full 64-bit boundary of the main accumulator, validating the complete carry chain width.

Test Procedure: We asserted the asynchronous hardware reset to flush all internal state, including the accumulator, the multi-cycle holding register, the FSM cycle counter, and all status flags. We then loaded the maximum positive 32-bit value via ADD_BIAS and scaled it to near the 64-bit ceiling using a MULT32 operation by the maximum unsigned scaling factor (0xFFFFFFFF). We verified the intermediate MULT32 result, then issued three successive ADD_BIAS commands of 0x7FFFFFFF followed by a final ADD_BIAS of 1 to precisely reach the maximum positive 64-bit value (0x7FFFFFFFFFFFFFFF). After verifying this value, we added 1 to force the rollover.

Expected Outcome: This proves the 64-bit adder propagates carry bits across its entire width without truncation or saturation. It confirms the accumulator cleanly transitions from the maximum positive value to the maximum negative value (0x8000000000000000), following standard two's complement wrap-around behavior. Unlike a simple signed zero-crossing test, this exercises every bit position in the carry chain. The use of a full asynchronous hardware reset at the start of this test was necessary because RST_ACC only clears the accumulator register and does not flush the internal holding register or FSM state, which was found to cause state contamination from prior MULT32 operations during verification.

Power Optimization Logic

- Multiply by Zero (Isolation)

Test Objective: To validate the power-saving operand isolation logic.

Test Procedure: We loaded a base value into the accumulator, then executed a MULT32 sequence with op_B explicitly tied to zero.

Expected Outcome: This ensures the operand isolation logic correctly zero out the inputs to the 32x8 adder tree when necessary, and the math resolves cleanly to zero without dropping bits.

- Isolation Sign Toggle

Test Objective: To confirm the operand isolation logic preserves sign integrity.

Test Procedure: We loaded a value of -1 (0xFFFFFFFF) into the accumulator and multiplied it by 1 using the MULT32 sequence.

Expected Outcome: This proves the operand isolation logic passes negative numbers cleanly. It ensures no sign-extension bits are accidentally zeroed out when the data moves from the multiplier array into the 32x8 adder tree.

Constrained Random Verification (CRV)

Automated Stress Testing

Test Objective: To validate the overall stability, mathematical correctness, and state synchronization of the ALU across a massive, unpredictable state space.

Test Procedure: Over a continuous 10000-cycle loop, the testbench generated randomized 5-bit instructions (excluding NOP) alongside randomized 32-bit operands for op_A and op_B.

Expected Outcome: This proves the hardware datapath, routing multiplexers, and multi-cycle FSM yield accurate mathematical results regardless of the sequence of commands or the magnitude of the data. It ensures the state machine maintains synchronization without dropping operations over an extended runtime.