Activation Block Microarchitecture

High-Level Overview

The Activation Block is the second and final stage of the PPU. It receives the Requantizer's 4/8-bit output, applies the activation function selected by act_fn_sel[2:0], and produces a 4/8-bit result on activation_out[7:0]. The block is purely combinational and contains no sequential elements.

The datapath is organized into two parallel precision paths - an INT8 path and an INT4 path - each containing its own set of activation function logic. A two-level input gating scheme ensures that the inactive path's inputs are forced to zero at all times, and that both paths are idle whenever EXEC_PP is not being executed. A final output multiplexer controlled by cfg_mode4x4 selects the active path's result onto activation_out.

All eight activation functions are implemented as synthesizable combinational logic. The four non-linear functions (Tanh, Sigmoid, GELU, SiLU) are logic-synthesized LUTs; the four linear functions (Identity, ReLU, Leaky ReLU α₁, Leaky ReLU α₂) use arithmetic logic.

Interface

| Port Name | Direction | Width | Description |
| --- | --- | --- | --- |
| activation_in | Input | 4/8-bit bus | Signed requantized value from the Requantizer. In INT8 mode the full byte is meaningful. In INT4 mode bits [3:0] hold the signed 4-bit result and bits [7:4] are don't-care. |
| cfg_mode4x4 | Input | 1 bit | Precision mode from the Controller's configuration register. High for INT4, low for INT8. |
| act_fn_sel | Input | 3-bit bus | Activation function select from the Controller's configuration register. Encoding defined in the PE Definition Document (Table 4). |
| ppu_en | Input | 1 bit | PPU enable from the Controller. Asserted for one cycle on EXEC_PP. Gates the Activation Block inputs to prevent combinational toggling during all other instructions. |
| activation_out | Input | 4/8-bit bus | Activated result. In INT8 mode the full byte holds the result. In INT4 mode bits [3:0] hold the result; bits [7:4] are don't-care. |

Table 1. Activation Block interface.

Block Diagram

Figure 1. Block-level microarchitecture.

Input Gating

The Activation Block implements a two-level input gating scheme.

The first level is global gating by ppu_en. When ppu_en is low - that is, every cycle except the EXEC_PP execution cycle - all inputs to both precision paths are forced to zero.

The second level is mode-based isolation. When ppu_en is high, only the path corresponding to the current precision mode receives the Requantizer's output; the other path's inputs remain forced to zero.

INT8 Path

The INT8 path evaluates the selected activation function on its gated input, interpreted as a signed 8-bit integer in the range [−128, +127]. The path contains two sub-blocks: a Linear Function Block and a Non-Linear LUT Bank, and a multiplexer.

Linear Function Block

This sub-block implements the four linear activation functions using combinational arithmetic logic. It receives the gated input and produces four 8-bit results.

Non-Linear LUT Bank

This sub-block implements the four non-linear activation functions as logic-synthesized LUTs. Each function maps all 256 possible 8-bit signed input values to their pre-computed 8-bit output values.

The four LUTs (Tanh, Sigmoid, GELU, and SiLU) receive the gated 8-bit input as their address and produce an 8-bit output.

INT8 Output Selection

A multiplexer controlled by act_fn_sel selects one of the eight results (four linear, four LUT) as the INT8 path's 8-bit output.

INT4 Path

The INT4 path evaluates the selected activation function on the gated 4-bit input, interpreted as a signed integer in the range [−8, +7]. The path mirrors the INT8 path's structure but operates on the reduced 4-bit address space.

Linear Function Block

All linear functions follow the same logic as their INT8 counterparts, operating on 4-bit values, but Leaky ReLU α₁, which is restricted to INT8 precision only, as stated in the PE Definition Document. The RTL produces a defined but architecturally irrelevant result for this encoding in INT4 mode.

Non-Linear LUT Bank

This sub-block follows the same logic as the INT8 counterpart, operating on 4-bit values.

INT4 Output Selection

A multiplexer controlled by act_fn_sel selects one of the eight results as the INT4 path's 4-bit output.

Output Multiplexer

A final multiplexer controlled by cfg_mode4x4 selects the active path's result onto activation_out. In INT8 mode, the full 8-bit result from the INT8 path is driven. In INT4 mode, the 4-bit result from the INT4 path occupies the lower four bits and the upper four bits are don't-care. The downstream output register captures the full byte, but the external system interprets only bits [3:0] in INT4 precision, as described in the PE Top-Level Microarchitecture document.

Reset Behavior

The Activation Block contains no sequential state. No reset action is required. All combinational outputs settle to deterministic values once the input signals are stable. On system reset, the Requantizer's output and the Controller's configuration signals are cleared to their default values, causing the Activation Block's combinational outputs to settle to the Identity function applied to a zero input, which is zero.