Instruction Set Justification: Mapping Project Requirements to PE Opcodes

Introduction

This document traces each opcode in the PE's Instruction Set Architecture back to a specific functional requirement defined in the project description. The goal is to demonstrate that every instruction exists for a concrete architectural reason rooted in the computational demands of quantized neural network inference, and that no unnecessary instructions were added.

The Inference Process

The ISA is explicitly built to execute the required pipeline for both INT4 and INT8 precisions.

iinputi×weighti+bias→rescale→factivation →output

To do this, the PE must accumulate dot-product partial sums across multiple cycles, add a bias, apply a scaling factor to convert the high-precision accumulator result back into a low-precision domain, and pass the scaled value through a non-linear activation function. Every opcode serves one stage of this pipeline or manages the datapath state between stages.

Opcode Justification

- NOP (00000)

A neural network accelerator does not receive a continuous, unbroken stream of valid instructions on every clock cycle. The external controller may need to stall the PE while waiting for data from memory, coordinating with other components if placed on an SoC, or pausing between independent inference passes. NOP guarantees the PE holds all internal state without corruption during these idle periods. It also enables automatic clock gating on the accumulator and datapath registers, reducing dynamic power consumption.

- RST_ACC (00001)

The PE processes neurons sequentially. Between these independent computations, the 64-bit accumulator must be cleared to zero. A full hardware reset would flush the entire PE state, including the configuration register that holds the current layer's precision and activation settings. RST_ACC provides a targeted, software-controlled clear that zeros the accumulator while preserving these critical layer configurations.

- MAC (00010)

The project description explicitly mandates both INT8 and INT4 multiply-accumulate support. Rather than dedicating two separate opcodes to these modes, the PE uses a single MAC instruction whose behavior is determined by the precision mode held in the configuration register. Adding a precision mode bit to this existing register costs only one extra flip-flop while eliminating a redundant opcode.

- ADD_BIAS (00011)

Standard quantized inference adds a per-neuron bias term (a single 32-bit signed value) directly to the accumulator after the dot product. Without this dedicated instruction, the only alternative would be encoding the bias addition as a MAC operation, which would require the user to construct complex, error-prone software schedules to force a 32-bit addition. ADD_BIAS provides a clean, direct path that avoids this complexity.

- SCALE (00100)

This instruction implements the rescaling multiplication that converts the accumulated dot product into the scaled domain required by the Post-Processing Unit (PPU). Because this operation cannot be performed in a single cycle using the existing 8x8 multiplier array without a prohibitive area cost, SCALE triggers an internal FSM that sequences the multiplication over several clock cycles. Additionally, it captures the PPU's shift amount from the lower 6 bits of operand_A. Attaching the shift amount to SCALE rather than the layer configuration allows the external controller to update it dynamically per sub-layer without issuing extra configuration instructions.

- LOAD_CFG (00101)

This instruction separates layer-wide configuration from per-cycle execution. Parameters like the arithmetic precision mode and the activation function remain constant across all neurons within a single layer. Instead of passing these same settings into every computation, LOAD_CFG loads them once at the layer boundary. It loads a single 4-bit value together upfront to configure the PE. The hardware retains this necessary context, freeing core execution instructions like MAC and SCALE to dedicate their operand buses entirely to actual data.

- EXEC_PP (00110)

EXEC_PP triggers the PPU to execute the final inference stages. It reads the accumulator and scales the value down using the pre-configured shift amount. It then references the current precision mode (INT4 or INT8) for two consecutive steps: first to apply the correct saturation bounds, and second to evaluate the selected activation function using the appropriate 4-bit or 8-bit logic. This produces the final quantized output.

- READ_ACC_BYTE (00111)

This instruction exists for testing and debug. Normally, the accumulator's only path to the output bus passes through the entire PPU chain. If a fault occurs, it is impossible to determine whether the MAC generated an incorrect dot-product or the PPU corrupted a valid result. READ_ACC_BYTE solves this observability problem by routing a specific byte of the 64-bit accumulator (selected by operand_A[2:0]) directly to the data_out bus. This guarantees precise fault isolation between the arithmetic datapath and the post-processing stages.

- READ_CFG (01000)

Like READ_ACC_BYTE, this instruction exists for testing and debug. It provides direct observability of the Processing Element's control state. While LOAD_CFG sets the layer's precision mode and activation function, there is normally no way to verify these settings without running a full inference pass. If the final quantized output is incorrect, a tester cannot easily determine if the configuration register loaded the wrong values or if the datapath math actually failed. READ_CFG solves this observability problem by routing the current configuration state directly to the output bus.

Architectural Exploration Cross-References

- Exploration Point B (RESCALE): The trade-off between an internal FSM and a RISC-like staged opcode sequence directly influenced the SCALE instruction. The selected internal FSM approach defines it as a single-trigger, multi-cycle instruction with busy/done handshaking.

- Exploration Point C (Activation): The trade-off between a ROM/RAM LUT and synthesizable combinational logic influenced the EXEC_PP and LOAD_CFG instructions. The selected logic-synthesized approach allows the activation function to execute the PPU pipeline without requiring dedicated instructions for memory resources.

Reserved Opcode Space

The 5-bit opcode encoding provides 32 possible instruction slots. Nine are currently defined, leaving 23 reserved for potential future expansion, such as INT16 operations or direct data movement instructions, without requiring a complete redesign of the Controller module.