Architectural Trade-Offs for the 8-Bit Activation Block

Introduction

The objective of this document is to define architectures and explore the trade-offs between them for the 8-bit Activation Block. This block requires a memory-based Look-Up Table (LUT) implementation and synthesizable logic to support complex non-linear functions. We have defined four potential architectural topologies for evaluation.

Architecture Options

This section details the architectural topologies evaluated for the Activation Block. In all configurations, the block is designed to support 6 distinct activation functions. Furthermore, to minimize dynamic power consumption, every configuration incorporates a global enable mechanism on its memory decoder. This ensures the memory array is only accessed and evaluated when a memory-based non-linear function is actively requested, preventing unnecessary toggling and power waste.

- Option A: SRAM (Flexible RAM)

This option utilizes a single SRAM block (256 rows × 8 bits). The RAM serves as a fully programmable LUT. Unlike the ROM options, the content is not fixed at fabrication; the user can load any arbitrary activation function into the memory, allowing for post-silicon updates or support for custom functions.

Trade-off: This architecture offers maximum flexibility. However, because it relies on a standard 6-transistor bitcell, it consumes significantly more area per bit than the ROM alternatives. It also suffers from high static power due to leakage across all rows and requires moderate dynamic power to precharge the differential bitlines during reads.

- Option B: ROM – Unified Memory

This option utilizes a single, monolithic ROM block (1024 rows × 8 bits) to store the four non-linear functions: Tanh, Sigmoid, GELU, and SiLU. To optimize memory usage, ReLU and Identity are not stored in the ROM. Instead, they are implemented via a parallel synthesizable logic path. The specific function is accessed by concatenating the activation selection bits with the input data to form the address in the unified block.

Trade-off: This setup avoids the complex output routing and multiplexing required by the banked architectures. However, it requires a complex 10-to-1024 decoder, which creates a massive area overhead. Furthermore, the long 1024-cell bitlines create high parasitic capacitance, making this option slower and more power-hungry during active reads compared to the split configurations.

- Option C: ROM Banked Memory Family

This approach physically divides the memory into four independent banks, where each bank is 256 rows by 8 bits. Each bank is dedicated to one of the four non-linear functions: Tanh, Sigmoid, GELU, and SiLU. Just like the Unified option, ReLU and Identity are offloaded to synthesizable logic.

Building on the general enable mechanism we established for the block, this architecture utilizes the activation selection signal as a specific enable mechanism for the individual banks. This ensures that only the specific ROM bank corresponding to the chosen activation is enabled. Because only the relevant bank is active, only the bitlines of that specific bank are charged and driven.

Option C1: Shared Decoder (Area Optimized)

To minimize area overhead, a global decoder is shared across all four banks. The decoded wordline is physically broadcasted to every bank.

Tradeoff: This configuration minimizes logic area and routing complexity. However, it consumes more dynamic power. Even though we only evaluate the bitlines of the active bank, broadcasting the wordline toggles the access transistors in the inactive banks. This wastes power on parasitic gate capacitance.

Option C2: Independent Decoders (Power Optimized)

Instead of a global decoder, this configuration duplicates the decoding logic so each bank gets its own dedicated local decoder. Building on the block's general enable mechanism, this setup uses the activation selection signal to activate only the specific local decoder required for the chosen function.

Trade-off: This completely isolates the inactive banks. Since the disabled decoders stay quiet, no wordlines toggle in the inactive banks, which heavily reduces dynamic power. The downside is a larger physical footprint due to the duplicated logic.

PPA and Complexity Summary

| Memory Type/ Feature | Option A: SRAM | Option B: ROM (Unified) | Option C1: Split ROM (Shared Decoder) | Option C2: Split ROM (Independent Decoders) |
| --- | --- | --- | --- | --- |
| Configuration | Single Block (256 rows × 8 bits) | Single Block (1024 rows × 8 bits) | 4 Small Banks (4 × [256 rows × 8 bits]) | 4 Small Banks (4 × [256 rows × 8 bits]) |
| Est. Transistors Count | ≈ 17,600 | ≈ 29,000 | ≈ 14,000 | ≈ 21,500 |
| Transistors per Bit | ≈ 8.6 (expensive) | ≈ 3.5 (moderate) | ≈ 1.7 (highly efficient) | ≈ 2.6 (efficient) |
| Decoder | Standard 8-to-256 | Complex 10-to-1024 | Shared 8-to-256 | Four Independent 8-to-256 |
| Dynamic Power (Read Only) | Moderate (Requires differential precharge of two bitlines per bit) | Medium-High (High parasitic capacitance due to long 1024-cell bitlines) | Low-Moderate (Bitlines isolated, but wordlines toggle in all banks wasting power) | Lowest (Both wordlines and bitlines toggle only in the active bank) |
| Static Power | High (leakage from 256 rows) | Low | Low | Low (Slightly higher leakage than C1 due to extra decoders) |
| Delay | Very Fast (uses sense amplifier) | Slow (high bitline capacitance) | Fast (low bitline capacitance) | Fast (low bitline capacitance, minimal prefix logic delay) |

Table 1. PPA and Complexity summary.

Transistor Count Analysis

The estimated transistor counts presented in Table 1 represent the aggregate of the storage core (bitcells) and the peripheral circuitry (overhead). The following breakdown details the methodology used to derive these estimates for each architecture.

Option A: SRAM (Single Block: 256 rows × 8 bits)

- Total Estimate: ~17,600 transistors

- Core (6T Bitcell): 12,288 (256 rows × 8 bits × 6 transistors/cell)

- Overhead: ~5,300

Includes: 8-to-256 Row Decoder, differential sense amplifiers, write drivers, and precharge circuits.

Option B: ROM – Unified Memory (Single Block: 1024 rows × 8 bits)

- Total Estimate: ~29,000 transistors

- Core (1T Bitcell): 8,192 (1024 rows × 8 bits × 1 transistor/cell)

- Overhead: ~20,800

Includes: Complex 10-to-1024 decoder and 1,024 individual wordline drivers. The overhead scales poorly, becoming roughly 2.5x larger than the core itself.

Option C: ROM – Banked Memory Family (4 Banks × 256 rows × 8 bits)

- Core (1T Bitcell): 8,192 (Equivalent capacity to Option B, spread across 4 banks).

- Option C1: Shared Decoder (Area Optimized)

  - Total Estimate: ~14,000 transistors

  - Overhead: ~5,800

Includes: A single shared 8-to-256 decoder, bank select multiplexers, and basic enable logic.

- Option C2: Independent Decoders (Power Optimized)

  - Total Estimate: ~21,500 transistors

  - Overhead: ~13,300

Includes: Four dedicated 8-to-256 decoders (adding roughly 7,500 transistors to the baseline overhead), prefix enable logic, and bank select multiplexers.

Physical Routing Trade-off

While logic area is minimized in the banked configurations, distributing the address bus to four separate macros and routing distinct output buses to a central multiplexer consumes additional metal layer area compared to a monolithic block. Option C2 adds further floorplan complexity due to the placement of the duplicated decoders.

Our Choice – WAITING FOR SYNTHESIS

Appendix – Activation Functions Mapping

| Activation Function | Logical Role & Behavior | Critical Application | Risk of Exclusion |
| --- | --- | --- | --- |
| ReLU (Rectified Linear Unit) | Pass-through / Zeroing | Computer Vision: The standard for image recognition (e.g., smart security sensors). Computationally efficient for deep networks. | PE would be inefficient and slow for standard image tasks. |
| Identity | Pass-through | Residual Learning: Crucial for "Skip Connections" in deep networks (e.g., ResNet) to prevent vanishing gradients, and for Linear Output Layers in regression tasks. | The PE would fail to support modern deep architectures (ResNets) or regression models, severely limiting its flexibility and accuracy on complex tasks. |
| Tanh (Hyperbolic Tangent) | Zero-Centered Mapping | Voice Processing: Essential for the "Cell State" in recurrent networks used in voice activation (e.g., smart toys). | PE would struggle to maintain long-term memory, failing at sequence tasks. |
| Sigmoid | Probability Switch | Required for "Forget/Update Gates". Decides what information to keep. | PE could not run standard LSTMs or GRUs without Sigmoid, breaking voice/audio applications. |
| GELU (Gaussian Error Linear Unit) | Probabilistic Smoothing | Modern AI (Transformers) | PE would be stuck supporting only older (pre-2018) models |
| SiLU (Sigmoid Weighted Linear Unit) | Self-Gating, allows small negative values to propagate | The standard activation for MobileNetV3 and EfficientNet, which are specifically designed for low-power edge devices | PE could not natively accelerate the most popular "mobile-optimized" networks, forcing them to fallback to less accurate approximations. |

Table 2. Activation functions.