Requantizer Microarchitecture

Figure 1. Block-level microarchitecture.

High-Level Overview

The Requantizer is the quantization and saturation stage of the PE datapath. It takes the full-precision 64-bit accumulator output from the ALU, applies a narithmetic right-shift with rounding to nearest integer, and clamps the result into a signed 8-bit (INT8) or 4-bit (INT4) integer.

The datapath is purely combinational and is organized into two blocks: Shift and Round, and Saturation Logic. Two internal registers store configuration state that is captured during the computation phase preceding requantization: a 6-bit shift-amount register loaded via a dedicated opcode from the lower bits of operand A, and a 1-bit mode flip-flop that implicitly captures the precision mode from the MAC4/MAC8 instruction stream.

Interface

| Port Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 bit | Main system clock. Internal registers capture their inputs on the positive edge. |
| rst | Input | 1 bit | Active-low asynchronous reset. Clears the shift amount register to zero, and the flip-flop to one. |
| accumulator | Input | 64-bit bus | The signed 64-bit accumulation result from the ALU. |
| shamt | Input | 6-bit bus | Load data for the shift-amount register, sourced from the lower 6 bits of operand A. |
| mode4x4 | Input | 1 bit | Precision mode signal from the Controller, reflecting the current MAC4/MAC8 operation. Captured into the internal flip-flop during MAC operations. |
| activation_in | Output | 8-bit bus | The final quantized, rounded, and saturated result. In INT8 mode, this is the direct 8-bit clamped value. In INT4 mode, the lower 4 bits hold the clamped result and the upper 4 bits are sign-extended. |

Shift and Round

This block takes the 64-bit accumulator input, shifts it right by the value held in the shift-amount register, and produces a rounded 8-bit intermediate result along with the raw shifted bit-vector for downstream overflow checking.

The 64-bit accumulator is first concatenated with a single zero bit at the LSB, forming a 65-bit signed value. This 65-bit value is then arithmetically right-shifted by the registered shamt value through a two-stage shifter. The coarse stage shifts by shamt[5:3] × 8 (0, 8, 16, …, 56 positions), implemented as a 65-bit-wide 8:1 multiplexer array controlled by the upper three bits of shamt. The fine stage shifts the coarse result by shamt[2:0] (0–7 positions), implemented as a second 65-bit-wide 8:1 multiplexer array controlled by the lower three bits. The output is the 65-bit signed signal fine_shifted.

Three fields are extracted from fine_shifted. Bit [0] is the round_bit. Bits [8:1] form pre_round_8bit, the 8-bit integer result before rounding. For INT4 mode, the relevant 4-bit result sits in pre_round_8bit[3:0].

A single 8-bit adder computes rounded_8bit = pre_round_8bit + round_bit. When the round bit is 0, the value passes through unchanged. When it is 1, the value is incremented by one, implementing rounding to nearest integer. For INT8 mode, the full 8-bit result is used. For INT4 mode, the rounded 4-bit result resides in rounded_8bit[3:0]. The signals fine_shifted, pre_round_8bit, and rounded_8bit are all forwarded to the Saturation Logic block.

Saturation Logic

This block determines whether the shifted and rounded result fits within the target signed range and, if not, forces the output to the appropriate saturation limit. It receives fine_shifted, pre_round_8bit, and rounded_8bit from the Shift and Round block, plus the original accumulator[63] sign bit and the registered mode4x4 value.

If a signed value fits within N bits after shifting, all bits above the N-bit result field must be uniform sign extension: all zeros for positive values, all ones for negative values. The module checks this property using reduction trees. For INT8, the 57 upper bits fine_shifted[64:8] are fed into two parallel reduction trees: a 57-input NOR tree (nor_tree_57, true if all 57 bits are zero) and a 57-input AND tree (and_tree_57, true if all 57 bits are one). If neither tree outputs true, the truncation overflow flag trunc_ovf_8 is asserted. For INT4, the check covers the 61 upper bits of fine_shifted[64:4]. The module cascades: it takes a 4-bit NOR/AND pair checks of fine_shifted[7:4] and the results are ANDed with the existing 57-bit tree outputs, to form nor_tree_61 and and_tree_61. The truncation overflow flag trunc_ovf_4 is derived from these cascaded results, reusing the 57-bit tree hardware for both modes.

Even when the pre-round value fits within range, incrementing by the round bit can push the result past the maximum positive boundary. The module detects this via the sign-toggle check: if the relevant MSB was 0 before rounding and 1 after, rounding overflowed. For INT8: round_ovf_8 = (~pre_round_8bit[7]) & rounded_8bit[7]. For INT4: round_ovf_4 = (~pre_round_8bit[3]) & rounded_8bit[3]. Negative values cannot overflow from rounding because incrementing moves them toward zero.

The two overflow sources are merged into a single pair of saturation flags per mode. The original input sign accumulator[63] determines the direction of truncation-based clamping. For INT8, the positive saturation flag is clamp_max_pos_8 = (trunc_ovf_8 & ~accumulator[63]) | round_ovf_8, asserting when the value was out-of-range and positive or when rounding overflowed. The negative saturation flag is clamp_min_neg_8 = (trunc_ovf_8 & accumulator[63]), asserting when the value was out-of-range and negative. For INT4, the conditions follow the same structure: clamp_max_pos_4 = (trunc_ovf_4 & ~accumulator[63]) | round_ovf_4 and clamp_min_neg_4 = (trunc_ovf_4 & accumulator[63]).

A priority multiplexer controlled by the registered mode4x4 selects the final 8-bit output. In INT8 mode: clamp_max_pos_8 forces the output to +127, otherwise clamp_min_neg_8 forces the output to −128, otherwise the output is the valid rounded_8bit. In INT4 mode: clamp_max_pos_4 forces the output to +7 sign-extended to 8 bits, otherwise clamp_min_neg_4 forces the output to −8 sign-extended to 8 bits, otherwise the lower 4 bits of rounded_8bit are sign-extended: {{4{rounded_8bit[3]}}, rounded_8bit[3:0]}.