Controller Unit Testing Documentation

Overview

The Controller is the instruction decoder and multi-cycle sequencer for the PE. It contains three sequential elements: a 5-bit opcode register, a 4-bit configuration register, and a 2-bit FSM. It also contains one single-bit status flip-flop (output_valid). All other control outputs are combinational functions of these registers, with the single exception of pe_ready, which is additionally gated by rst directly. The verification strategy uses directed testing organized into seven groups to cover every architectural path, followed by a constrained random phase that validates structural invariants over 10,000 cycles. Expected values are derived from a golden reference model that runs in lockstep with the DUT on every clock cycle.

Golden Reference Model

The testbench contains a cycle-accurate golden reference model implemented as two cooperating tasks: model_update_sequential and model_compute_outputs. The model maintains internal copies of all four Controller registers (opcode_reg, config_reg, FSM state, output_valid_reg) and computes expected values for all 20 outputs on every clock edge.

Sequential Update

On each rising clock edge, the model evaluates all register inputs from the OLD (pre-edge) state, then commits the new values. The update order within a single edge is:

- Compute the capture gate from the OLD pe_ready: capture_gate = exp_pe_ready AND opcode_valid AND chp_slct.

- Compute the new opcode register: opcode_reg_new = capture_gate ? pe_opcode : NOP.

- Validate the new opcode register through a case map. Recognized encodings pass through; all others map to NOP.

- Evaluate the configuration register input from the OLD validated_cmd: config_reg captures cfg_data_in if validated_cmd_old = LOAD_CFG. There is no FSM-state guard on this capture, matching the RTL — opcode_reg cannot become LOAD_CFG outside IDLE because pe_ready is low for all non-IDLE FSM states.

- Evaluate the output_valid flip-flop input from the OLD outputs: output_valid_reg captures (ppu_en_old OR read_acc_en_old OR read_cfg_en_old).

- Evaluate the FSM next-state from the OLD state and OLD validated_cmd per the FSM transition table.

- Commit: all model registers advance to their new values.

The critical discipline is that all register inputs are read from pre-edge model state. The opcode register is the only register whose new value depends on the current cycle's external inputs (pe_opcode, opcode_valid, chp_slct). All other registers depend exclusively on the previous cycle's internal state. Violating this ordering — for example, computing the FSM transition from the newly captured validated_cmd rather than the old one — would shift the model's timing by one cycle and cause failures on every SCALE and LOAD_CFG test.

Combinational Output Computation

After the sequential commit, the model computes all 20 expected outputs from the committed (post-edge) state. The computation uses the same Boolean expressions as the RTL Instruction Decoder section, with the seven IDLE-gated command predicates (cmd_is_rst_acc, cmd_is_mac, cmd_is_bias, cmd_is_scale0, cmd_is_exec_pp, cmd_is_read_acc, cmd_is_read_cfg) and the three SCALE-state predicates (sc1, sc2, sc3) feeding the output assignments:

- mode4x4 = ~any_scale AND ~config_reg[3], where any_scale = cmd_is_scale0 OR sc1 OR sc2 OR sc3.

- a_is_signed = cmd_is_mac OR sc3; b_is_signed = cmd_is_mac.

- scale_mode = any_scale; scale_phase[1] = sc2 OR sc3; scale_phase[0] = sc1 OR sc3.

- mac4_en = cmd_is_mac AND ~config_reg[3]; mac8_en = cmd_is_mac AND config_reg[3].

- acc_source[1] = any_scale OR cmd_is_bias; acc_source[0] = (cmd_is_mac AND config_reg[3]) OR cmd_is_bias.

- acc_wr_en = cmd_is_mac OR cmd_is_bias OR any_scale.

- latch_en = shamt_wr_en = acc_load = cmd_is_scale0 (single physical output exposed as three named ports).

- rst_acc = cmd_is_rst_acc; ppu_en = cmd_is_exec_pp; read_acc_en = cmd_is_read_acc; read_cfg_en = cmd_is_read_cfg.

- act_fn_sel = config_reg[2:0]; config_out = config_reg[3:0].

- pe_ready = rst AND (next_state = IDLE) AND (opcode_reg ≠ SCALE).

- output_valid = output_valid_reg.

After computing the expected outputs, the model also stores ppu_en, read_acc_en, and read_cfg_en in three "previous output" variables for use as the output_valid_reg input on the next cycle.

Stimulus Tasks

The testbench provides four stimulus tasks built on top of the model evaluator:

- send_and_check(opcode, valid, cs, cfg, name) drives the four DUT inputs, waits for the next rising clock edge plus 1 ns for combinational settling, and then calls model_update_sequential and model_compute_outputs followed by a full 20-output comparison via check_all_outputs.

- wait_and_check(name, hold_cfg) is a thin wrapper that issues a NOP with both qualifiers low. Used to advance the FSM unconditionally during multi-cycle SCALE operation and during drain cycles.

- issue_load_cfg(cfg_val, name) is a two-cycle macro: the first cycle drives LOAD_CFG with valid qualifiers, capturing it into opcode_reg; the second cycle drives NOP, allowing config_reg to capture cfg_data_in (which was held stable across both cycles). Both cycles are checked against the model.

- apply_reset() drives rst low for two clock periods, resets the model state, then releases rst and advances one edge so pe_ready re-enters its post-reset value.

Test Plan

Group 1: Asynchronous Reset

Test 1.1: Power-on reset state. Test objective: Verify that all status outputs are at their post-reset values after apply_reset. Test procedure: Call apply_reset and check pe_ready, config_out, and output_valid. Expected outcome: pe_ready = 1, config_out = 4'b0000, output_valid = 0. The full set of 20 outputs is verified by the model on the post-reset edge.

Test 1.2a: Asynchronous pe_ready drop. Test objective: Verify that pe_ready drops asynchronously when rst is asserted mid-operation. Test procedure: Issue SCALE, advance one cycle into SC1, then drive rst low and check pe_ready 1 ns later (combinationally, before any clock edge). Expected outcome: pe_ready = 0.

Test 1.2b: Mid-operation reset cleanup. Test objective: Verify that asynchronous reset asserted during SCALE leaves no residual state. Test procedure: After 1.2a, call apply_reset and issue a NOP. Expected outcome: pe_ready = 1 and output_valid = 0 — no residual SCALE-related signaling, FSM cleanly returned to IDLE, opcode register cleared.

Test 1.3: Reset clears configuration. Test objective: Verify that asynchronous reset clears config_reg even after a successful LOAD_CFG. Test procedure: Issue LOAD_CFG with cfg_data_in = 4'b1101, then apply_reset, then check config_out. Expected outcome: config_out = 4'b0000.

Group 2: Opcode Gating

Test 2.1: Normal capture. Test objective: Verify that an opcode with all qualifiers high is captured and decoded. Test procedure: Issue MAC with opcode_valid = 1, chp_slct = 1, after a fresh reset (so pe_ready = 1). Expected outcome: mac4_en = 1, mac8_en = 0 (since config_reg[3] = 0 after reset).

Test 2.2: opcode_valid low gates capture. Test objective: Verify that a low opcode_valid suppresses opcode capture. Test procedure: Issue MAC with opcode_valid = 0. Expected outcome: Both mac4_en and mac8_en are zero — the opcode register held NOP rather than capturing MAC.

Test 2.3: chp_slct low gates capture. Test objective: Verify that a low chip-select suppresses opcode capture. Test procedure: Issue MAC with chp_slct = 0. Expected outcome: Both mac4_en and mac8_en are zero.

Test 2.4: pe_ready low gates capture. Test objective: Verify that during SCALE multi-cycle operation, an attempted MAC injection is silently dropped. Test procedure: Issue SCALE (FSM transitions to SC1), then issue MAC on the next cycle. Expected outcome: Both mac4_en and mac8_en are zero, scale_mode = 1 (the FSM continues its SCALE sequence undisturbed). The remaining SC2, SC3, and drain cycles are advanced with wait_and_check.

Test 2.5: All qualifiers low. Test objective: Verify the conjunctive gating semantics with all qualifiers low. Test procedure: Issue MAC with opcode_valid = 0, chp_slct = 0. Expected outcome: Both mac4_en and mac8_en are zero.

Test 2.6: Invalid encoding maps to NOP. Test objective: Verify that an unrecognized opcode is decoded as NOP. Test procedure: Issue 5'b11111 (out of the 9-opcode space) with all qualifiers high. Expected outcome: acc_wr_en = 0 — no datapath activity is triggered. The full output vector is also model-checked.

Group 3: Single-Cycle Truth Table

This group issues each single-cycle command from a fresh reset state. Every test is checked two ways: the full 20-output comparison against the golden model runs automatically as part of send_and_check, and an additional explicit if-statement asserts the headline output(s) that define the architectural intent of the command.

Test 3.1: NOP. Issued from reset; full output vector model-checked.

Test 3.2: RST_ACC. Issue RST_ACC. Expected: rst_acc = 1.

Test 3.3: MAC in INT4 mode. Load configuration 4'b0000 (INT4, Identity), then issue MAC. Expected: mode4x4 = 1, acc_source = 2'b00, mac4_en = 1, mac8_en = 0, a_is_signed = 1, b_is_signed = 1.

Test 3.4: MAC in INT8 mode. Load configuration 4'b1000 (INT8, Identity), then issue MAC. Expected: mode4x4 = 0, acc_source = 2'b01, mac4_en = 0, mac8_en = 1.

Test 3.5: ADD_BIAS. Issue ADD_BIAS. Expected: acc_source = 2'b11, acc_wr_en = 1.

Test 3.6: EXEC_PP. Issue EXEC_PP. Expected: ppu_en = 1, acc_wr_en = 0.

Test 3.7: READ_ACC_BYTE. Issue READ_ACC_BYTE. Expected: read_acc_en = 1, acc_wr_en = 0.

Test 3.8: READ_CFG. Issue READ_CFG. Expected: read_cfg_en = 1, acc_wr_en = 0.

Test 3.9: LOAD_CFG. Issue LOAD_CFG with cfg_data_in = 4'b1010 via the two-cycle issue_load_cfg macro. Expected: config_out = 4'b1010.

Group 4: Configuration Register

Test 4.1: Exhaustive 4-bit load. Test objective: Verify that all 16 possible configuration values are captured correctly. Test procedure: For each value 0..15, apply reset, issue LOAD_CFG with that value, check config_out. Expected outcome: config_out matches the loaded value for every iteration.

Test 4.2: Persistence across non-LOAD_CFG instructions. Test objective: Verify that config_reg holds its value across unrelated commands. Test procedure: Load 4'b1101, then issue MAC, ADD_BIAS, RST_ACC; check config_out. Expected outcome: config_out = 4'b1101.

Test 4.3: Overwrite by second LOAD_CFG. Test objective: Verify that a second LOAD_CFG overwrites the first. Test procedure: Issue LOAD_CFG with 4'b1010, then LOAD_CFG with 4'b0101. Expected outcome: config_out = 4'b0101.

Test 4.4: mode4x4 inversion in IDLE/NOP context. Test objective: Verify that mode4x4 reflects ~config_reg[3] when no SCALE cycle is active. Test procedure: Load 4'b0000 and check mode4x4 = 1 (post-LOAD_CFG capture cycle is NOP, so any_scale = 0); then load 4'b1000 and check mode4x4 = 0. Expected outcome: mode4x4 inverts with config_reg[3] outside SCALE cycles.

Test 4.5: Configuration switch propagates to MAC decode. Test objective: Verify that a configuration change propagates correctly to MAC-mode decode signals. Test procedure: Load 4'b0000, issue MAC, check INT4 decode (mode4x4 = 1, acc_source = 2'b00, mac4_en = 1); then load 4'b1000, issue MAC, check INT8 decode (mode4x4 = 0, acc_source = 2'b01, mac8_en = 1). Expected outcome: All checks pass.

Group 5: SCALE Full Sequence

Test 5.1: Per-cycle signal vector. Test objective: Verify the signal vector on each of the four SCALE execution cycles. Test procedure: Issue SCALE from a fresh reset and check the cycle-0 vector; then advance through SC1, SC2, SC3 with wait_and_check and verify the per-cycle signal expectations. Expected outcomes:

- Cycle 0: scale_mode = 1, scale_phase = 2'b00, latch_en = 1, acc_load = 1, shamt_wr_en = 1, a_is_signed = 0, b_is_signed = 0, acc_source = 2'b10, acc_wr_en = 1.

- SC1: scale_phase = 2'b01, latch_en = 0, acc_load = 0, shamt_wr_en = 0, a_is_signed = 0.

- SC2: scale_phase = 2'b10, a_is_signed = 0.

- SC3: scale_phase = 2'b11, a_is_signed = 1 (the sign-flip cycle for the upper-byte multiplication).

A drain cycle wait_and_check follows SC3 to return the FSM to IDLE-with-pe_ready-high.

Test 5.2: pe_ready drop and rise. Test objective: Verify pe_ready timing across a SCALE sequence. Test procedure: Issue SCALE and check pe_ready = 0 on the same cycle (cycle 0, FSM still in IDLE but opcode_reg = SCALE — the (opcode_reg ≠ SCALE) term in pe_ready is zero). Advance through SC1, SC2, and SC3, and check pe_ready = 1 on the SC3 cycle. Expected outcome: pe_ready = 0 during cycle 0 and during SC1–SC2, and rises to 1 on SC3.

Test 5.3: Injection rejection during SC1 and SC2. Test objective: Verify that opcodes issued during SC1 and SC2 are silently dropped. Test procedure: Issue SCALE, then attempt MAC on SC1 (check both isolation enables zero), ADD_BIAS on SC2 (no explicit assertion; model-checked), advance through SC3, and check that the SCALE FSM is still progressing normally on SC3 (scale_mode = 1, scale_phase = 2'b11). Expected outcome: The MAC and ADD_BIAS injections are rejected; the FSM advances through its full sequence undisturbed.

Test 5.4: Back-to-back SCALE sequences. Test objective: Verify that two SCALE sequences can be issued back-to-back with no idle cycle between them, exercising the predictive pe_ready assertion at SC3. Test procedure: Issue SCALE, advance through SC1, SC2, SC3; on the SC3 cycle issue the second SCALE (which is captured on the SC3 → IDLE edge and decoded on the following cycle as cycle 0 of the second sequence); advance the second sequence through SC1, SC2, SC3 and one final cycle. Expected outcome: The second SCALE produces scale_mode = 1, scale_phase = 2'b00, latch_en = 1, acc_load = 1 on the cycle immediately following the first sequence's SC3, with no gap.

Group 6: output_valid Generation

Test 6.1: EXEC_PP single pulse. Test objective: Verify that output_valid pulses for exactly one cycle, one cycle after EXEC_PP. Test procedure: Issue EXEC_PP and check output_valid = 0 (not yet pulsed); advance one cycle (NOP) and check output_valid = 1; advance another cycle (NOP) and check output_valid = 0. Expected outcome: Exactly one pulse, one cycle after EXEC_PP.

Test 6.2: READ_ACC_BYTE single pulse. Same as 6.1 with READ_ACC_BYTE.

Test 6.3: READ_CFG single pulse. Same as 6.1 with READ_CFG.

Test 6.4: No spurious pulse. Test objective: Verify that non-output-producing commands do not generate output_valid. Test procedure: Issue MAC and a NOP, check output_valid = 0; then RST_ACC and a NOP, check output_valid = 0. Expected outcome: output_valid stays low.

Test 6.5: Back-to-back output-producing commands. Test objective: Verify that two consecutive output-producing commands generate two consecutive output_valid pulses. Test procedure: Issue EXEC_PP, then READ_ACC_BYTE (output_valid pulses for the EXEC_PP); advance one NOP (output_valid pulses for the READ_ACC_BYTE); advance another NOP (output_valid drops). Expected outcome: Two consecutive cycles of output_valid = 1, then a drop.

Group 7: Instruction Sequences

Test 7.1: Full inference pipeline. Test objective: Verify a representative end-to-end instruction sequence. Test procedure: Issue LOAD_CFG with 4'b1011 (INT8, custom activation), then three MACs, an ADD_BIAS, a SCALE (advance through SC1, SC2, SC3), an EXEC_PP issued on the SC3 → IDLE edge, and a NOP. Check output_valid on the cycle following EXEC_PP. Expected outcome: output_valid = 1 — full pipeline produces a final result indication.

Test 7.2: Layer transition. Test objective: Verify a configuration change between two layers of computation. Test procedure: Load configuration 4'b1000 (INT8), perform a full inference (MAC, ADD_BIAS, SCALE with full FSM advance, EXEC_PP, NOP), then RST_ACC, then load configuration 4'b0110 (INT4, custom activation), then MAC. Check the post-transition MAC decode. Expected outcome: After the second LOAD_CFG and MAC, mode4x4 = 1, acc_source = 2'b00, mac4_en = 1 — confirming that the configuration change propagates correctly to the next inference layer.

Test 7.3: Debug readback sequence. Test objective: Verify that READ_ACC_BYTE and READ_CFG do not disturb the accumulator write enable or any other MAC & SCALE datapath signal. Test procedure: Issue two MACs, then READ_ACC_BYTE, then another MAC, then SCALE → advance through SC1, SC2, SC3 → EXEC_PP, NOP, then READ_CFG. Check acc_wr_en on both READ_ACC_BYTE and READ_CFG cycles. Expected outcome: acc_wr_en = 0 during both READ_ACC_BYTE and READ_CFG; the surrounding MAC and SCALE operations execute normally as verified by the full model comparison on every cycle.

Group 8: Constrained Random Verification

Test 8.1: 10,000-cycle random stimulus. Test objective: Exercise arbitrary opcode interleavings, qualifier toggling, configuration changes, and invalid encodings to expose unexpected state interactions not covered by directed tests. Test procedure: Over a loop of 10,000 iterations, the testbench generates a random opcode (70% weighted toward the nine valid encodings, 30% arbitrary 5-bit values), random opcode_valid (85% high), random chp_slct (90% high), and a random 4-bit cfg_data_in. On every cycle, the golden reference model advances in lockstep and all 20 outputs are compared. Additionally, four structural invariants are checked:

- Operand enables and scale_mode are mutually exclusive: (mac4_en OR mac8_en) AND scale_mode is never high (a MAC and a SCALE cycle cannot be simultaneously active).

- mac4_en and mac8_en are mutually exclusive: the two precision-isolation enables are never both high (only one MAC path is selected at a time).

- The output multiplexer's select is preserved: at most one of ppu_en, read_acc_en, and read_cfg_en is high at any given time.

- acc_load is a SCALE-only signal: acc_load is never high when scale_mode is low (acc_load is valid only during SCALE cycle 0).

Expected outcome: All 20 output comparisons pass on every cycle. No structural invariant violations occur. This validates the Controller's resilience to arbitrary input sequences including mid-sequence configuration changes, chp_slct toggling during and between instructions, invalid encodings mixed with valid ones, and the FSM's immunity to input noise during multi-cycle operation.