IO Block Unit Testing Documentation

Overview

The IO Block is a sequential module with three registers (operand_A_reg, operand_B_reg, output register) and two combinational mux fabrics (byte-select 8:1 and output 3:1). Verification must therefore exercise both temporal behaviors (capture conditions, hold, async reset) and combinational behaviors (mux source selection, data-side isolation, write-enable composition). The strategy combines directed tests for every architectural feature and named edge condition, followed by long-running constrained-random stress against a SystemVerilog golden reference.

Golden Reference Model

The reference is a cycle-accurate SV model that mirrors the IO Block's update equations. On every rising clock edge it executes:

if (!rst) { op_A_m, op_B_m, dout_m = 0 }

else {

if (valid_A & pe_ready & chp_slct) op_A_m <= operand_A

if (valid_B & pe_ready & chp_slct) op_B_m <= operand_B

byte_mux_m  = read_acc_en ? accumulator[8*op_A_m[2:0] +: 8] : 8'b0

out_src_m   = read_acc_en ? byte_mux_m

: read_cfg_en ? {4'b0, config_out}

:               activation_out

if (ppu_en | read_acc_en | read_cfg_en) dout_m <= out_src_m

}

The model uses the same registered-then-combinational ordering as the DUT to keep race-free comparisons. Comparisons use === to handle X-propagation cleanly.

Directed Test Groups

Group 1: Asynchronous Reset

Test 1.1: Power-on reset. Hold rst = 0 for two clock periods, deassert. Verify operand_A_reg, operand_B_reg, data_out are all zero after release.

Test 1.2: Mid-operation reset. Drive operand captures and an output_wr_en cycle to populate all three registers, then assert rst = 0 asynchronously between clock edges. Verify all three registers clear immediately, independent of clock phase.

Test 1.3: Reset hold. Hold rst = 0 while toggling all data inputs and control signals randomly. Verify outputs remain zero throughout — confirms the async reset cone dominates every register.

Group 2: Input Register Bank: Capture Conditions

Test 2.1: valid_A × pe_ready × chp_slct truth table. Sweep all 8 combinations of {valid_A, pe_ready, chp_slct} while driving a unique pattern on operand_A each cycle. Only the all-ones case must update operand_A_reg; all other 7 combinations must hold the previous value. Repeat the same 8-case sweep against valid_B / chp_slct / pe_ready for operand_B_reg.

Test 2.2: Channel independence. Drive valid_A=1, valid_B=0 for several cycles with both operand buses changing, then flip to valid_A=0, valid_B=1. Verify A captures only in the first phase and B only in the second — no cross-coupling.

Test 2.3: Continuous streaming. With pe_ready & chp_slct = 1 and valid_A = valid_B = 1 for 32 consecutive cycles, drive an incrementing pattern on both buses. Verify each register captures the correct value on every edge.

Test 2.4: Mid-stream gating. Stream operands as in 2.3, then deassert chp_slct for 5 cycles while operands continue to change. Verify both registers hold the last pre-deassertion value across the gap, then resume capture cleanly when chp_slct returns.

Group 3: Byte-Select Multiplexer: Encoding

Test 3.1: All 8 select positions. Load operand_A_reg = 32'h0000_0000 then ..._0001 … ..._0007 over successive cycles (with valid_A=1). Drive accumulator = 64'hFEDC_BA98_7654_3210 and read_acc_en = 1, ppu_en = 0, read_cfg_en = 0. Verify data_out captures 0x10, 0x32, 0x54, 0x76, 0x98, 0xBA, 0xDC, 0xFE respectively. Confirms the select encoding 0→bits[7:0] … 7→bits[63:56].

Group 4: Byte-Select Path: Data-Side Isolation

Test 4.1: Isolation under select toggling. Load operand_A_reg with a known index. Hold read_acc_en = 0 for many cycles while toggling accumulator randomly and re-loading operand_A_reg with all 8 indices. Drive read_cfg_en = 0, ppu_en = 0. Verify data_out stays at its last captured value (no output_wr_en should fire) and that the byte-mux fanin to the output mux contributes zero.

Test 4.2: Isolation handover. With accumulator = 64'hFFFF_FFFF_FFFF_FFFF and read_acc_en = 0, observe data_out unchanged. Pulse read_acc_en = 1 for one cycle. Verify data_out updates to the selected byte (0xFF). Confirms isolation gate releases cleanly on assertion edge.

Group 5: Output Multiplexer: Source Selection

Test 5.1: Default (activation) path. Drive read_acc_en = read_cfg_en = 0, ppu_en = 1. Sweep activation_out over 16 distinct values. Verify data_out captures each one exactly.

Test 5.2: read_acc_en path. With a pre-loaded operand_A_reg index and a known accumulator, assert read_acc_en = 1, read_cfg_en = 0, ppu_en = 0. Verify the output mux selects byte_mux_out and the captured data_out matches the indexed accumulator byte. Concurrently drive activation_out to a non-matching value to confirm the override.

Test 5.3: read_cfg_en path. Drive config_out = 4'hA, read_cfg_en = 1, all other enables zero. Verify data_out captures 8'h0A — confirms the upper 4 bits are zero-padded by the IO Block (not driven from config_out).

Test 5.4: Source override under contention. Drive activation_out = 8'hAA continuously. Pulse read_acc_en for one cycle (with valid byte-mux input), then read_cfg_en for one cycle. Verify data_out tracks the override sources on those cycles and returns to capturing activation_out on subsequent ppu_en cycles.

Group 6: Output Register: Write-Enable Composition

Test 6.1: Each enable individually. With three separate cycles, assert exactly one of {ppu_en, read_acc_en, read_cfg_en} per cycle (consistent with the matching source). Verify data_out updates each time.

Test 6.2: No-enable hold. After populating data_out with a known value, drive all three enables to zero for 16 cycles while the source-mux inputs (activation_out, accumulator, config_out) toggle randomly. Verify data_out holds its previous value across all 16 cycles.

Test 6.3: Reset during hold. Repeat 6.2 then assert rst = 0 mid-hold. Verify clear to zero. Confirms reset path is independent of output_wr_en.

Group 7: Protocol Violation Surveillance

The IO Block trusts the Controller for read_acc_en and read_cfg_en mutual exclusivity. The testbench installs SystemVerilog assertions (concurrent, on the IO Block boundary) to catch any violation:

assert property (@(posedge clk) disable iff (!rst)

!(read_acc_en && read_cfg_en))

else $error("Protocol violation: simultaneous read enables");

A second assertion guards the valid × pe_ready × chp_slct combination — these are not strictly mutually-exclusive but the assertion logs every capture event so coverage tooling can track per-condition hit counts.

Group 8: Mode-Agnostic Output Propagation

Test 8.1: INT4 sign-extended pattern. Drive activation_out with PPU-style sign-extended INT4 patterns (8'hF8, 8'h07, 8'hFC, 8'h03) on ppu_en cycles. Verify data_out captures all 8 bits unchanged. Confirms the IO Block is mode-agnostic and does not mask the upper nibble — the PPU owns the sign extension.

Test 8.2: INT8 full-range. Drive activation_out with 8'h00, 8'h7F, 8'h80, 8'hFF. Verify pass-through.

Group 9: Cross-Functional Sequences

Test 9.1: EXEC_PP → READ_ACC_BYTE → READ_CFG. Drive a three-cycle sequence emulating the Controller's typical post-MAC drain: ppu_en pulse, then read_acc_en pulse with a freshly-loaded operand_A_reg[2:0], then read_cfg_en pulse. Verify data_out updates correctly on each cycle from the corresponding source.

Test 9.2: Operand capture during output read. Concurrently assert valid_A = 1 (loading a new operand_A) and read_acc_en = 1 in the same cycle. The read_acc_en cycle uses the previously-captured operand_A_reg[2:0] (the new write happens on the same edge as the data_out capture, so the byte-mux sees the old value). Verify this ordering matches the reference model.

Test 9.3: Mid-read reset. Issue read_acc_en then assert rst at the same edge. Verify all registers clear and no spurious data_out value latches.

Group 10: Constrained Random Stress (10,000 cycles)

Per-cycle stimulus generation:

| Signal | Distribution |
| --- | --- |
| operand_A, operand_B | Uniform $urandom (full 32-bit range) |
| accumulator | Uniform $urandom (full 64-bit range) |
| activation_out, config_out | Uniform |
| valid_A, valid_B, chp_slct, pe_ready | 80% asserted (high-traffic operand pipeline) |
| ppu_en, read_acc_en, read_cfg_en | One-hot among 4 states {none, ppu, racc, rcfg} with 70% none / 10% each enable (matches Controller traffic profile) |
| rst | 1% per-cycle pulse, held for 1–3 cycles |

The reference model runs in lockstep. Every cycle, all three outputs (operand_A_reg, operand_B_reg, data_out) are compared with ===. Failures are logged with the cycle index and full input vector for replay.

The 10,000-cycle target produces approximately 1,000 hits per output-mux source and ~1,250 per byte-select index — sufficient for statistical confidence on every architectural path. The 1% reset rate guarantees ~100 reset-recovery sequences in the run.

Coverage Targets (covergroup)

- byte_select_cp — operand_A_reg[2:0] ∈ {0…7}, sampled when read_acc_en = 1. Goal: 100% bins.

- output_source_cp — {ppu_en, read_acc_en, read_cfg_en} × op_A_reg[2:0]. Goal: all 4 source states × all 8 indices ≥ 1 hit.

- capture_cp — {valid_A, valid_B, pe_ready, chp_slct} per channel. Goal: all 16 bins ≥ 1 hit per channel.

- reset_cross_cp — reset asserted while each of {operand_capture, output_write, isolation_gated} is in progress. Goal: all 3 crosses hit.