Controller Unit Testing Documentation

Overview

The MAC & SCALE module is the PE's arithmetic datapath. The module is sequential and multi-mode: it supports single-cycle MAC (INT4 and INT8), single-cycle ADD_BIAS, and a multi-cycle SCALE operation. The verification strategy combines exhaustive directed tests targeting every architectural path with a constrained-random self-checking flow that propagates a software shadow accumulator across thousands of mixed operations. Expected values are derived from algebraic golden reference functions that specify the intended computation.

Golden Reference Model

Three functions compute the mathematically correct output for any input combination.

golden_mac8(A, B) computes the sum of four signed 8×8 products into an 18-bit signed intermediate, sign-extended to 32 bits:

S=i=03signedA8i+7:8i×signed(B[8i+7:8i])

golden_mac4(A, B) computes the sum of eight signed 4×4 products into an 11-bit signed intermediate, sign-extended to 32 bits:

S=i=07signedA4i+3:4i×signed(B[4i+3:4i])

golden_scale(acc_lo, B) computes the full 64-bit signed × unsigned product:

R = signedacclo×unsigned(B)

A software shadow accumulator tracks the expected accumulator state across an arbitrary mix of operations. Its lower 32 bits are updated by the appropriate golden function on every MAC or BIAS, and replaced wholesale with the 64-bit golden_scale result on SCALE.

Test Plan

Group 1: Reset and Hold

Test 1.1: Asynchronous reset. Test objective: Verify that asserting rst clears all internal sequential state to a known value. Test procedure: We asserted rst for two clock cycles, deasserted it, and inspected the accumulator after the next negedge. Expected outcome: accumulator equals 64'b0. This confirms the asynchronous clear path reaches the 64-bit accumulator register.

Test 1.2: Hold during idle. Test objective: Verify that with acc_wr_en low and rst_acc low, the accumulator preserves its value indefinitely. Test procedure: After reset, we held all control signals inert for five clock cycles. Expected outcome: accumulator remains zero across all five cycles. This validates the write-enable gating in the accumulator capture logic.

Test 1.3: Synchronous accumulator clear. Test objective: Verify that rst_acc synchronously clears the accumulator on the next clock edge, independently of the asynchronous reset path. Test procedure: We preloaded the accumulator with a non-zero MAC8 result, then asserted rst_acc for a single cycle. Expected outcome: accumulator becomes zero on the rising edge while rst_acc is high. This confirms the synchronous reset takes priority over acc_wr_en.

Group 2: MAC8 Directed Tests

Test 2.1: Zero operand. Test objective: Verify that a zero operand on either input produces a zero contribution. Test procedure: We applied A=0x00000000 with B=0xDEADBEEF, and the symmetric case A=0xDEADBEEF with B=0x00000000. Expected outcome: accumulator[31:0] = 0 in both cases. This proves the multiplier array correctly produces zero for zero operands and the reduction tree correctly sums to zero.

Test 2.2: Identity sum. Test objective: Verify the simplest non-trivial MAC8 result. Test procedure: We applied A=B=0x01010101 (four bytes of value 1). Expected outcome: accumulator[31:0] = 4. This is the canonical sanity check that the reduction tree sums all four 8×8 products.

Test 2.3: Maximum positive magnitude. Test objective: Stress the multiplier array and reduction tree with the largest positive signed inputs. Test procedure: We applied A=B=0x7F7F7F7F (four bytes of +127). Expected outcome: accumulator[31:0] = 4 · 127² = 64516. This exercises the upper width of the 16-bit product and the 18-bit reduction.

Test 2.4: Maximum negative magnitude. Test objective: Stress the array with the largest negative signed inputs (the asymmetric two's-complement boundary). Test procedure: We applied A=B=0x80808080 (four bytes of −128). Expected outcome: accumulator[31:0] = 4 · 16384 = 65536. The product of two minimum-negative signed values is the maximum positive 16-bit value (+16384), validating two's-complement multiplication at its asymmetric boundary.

Test 2.5: Sequential accumulation. Test objective: Verify that successive MAC8 results add into the running accumulator rather than overwriting it. Test procedure: We issued four consecutive identity MAC8 operations. Expected outcome: accumulator[31:0] = 16 (4 + 4 + 4 + 4). This validates the accumulator feedback path and the lower 32-bit adder.

Test 2.6: Positive/negative cancellation. Test objective: Verify cancellation behavior across operations of opposite signs. Test procedure: We performed a maximum-positive MAC followed by a mixed-sign MAC of equal-magnitude opposite contribution. Expected outcome: accumulator[31:0] equals the algebraic sum of the two golden_mac8 results. This stresses signed addition at non-trivial intermediate widths.

Group 3: MAC4 Directed Tests

Test 3.1: Zero operand. Test objective: Verify zero behavior in INT4 mode. Test procedure: A=0x00000000, B=0x12345678. Expected outcome: accumulator[31:0] = 0. Validates the INT4 multiplier array and 8-product reduction tree.

Test 3.2: Identity sum. Test objective: Verify the canonical eight-product reduction. Test procedure: A=B=0x11111111 (eight nibbles of value 1). Expected outcome: accumulator[31:0] = 8. This sanity-checks the three-stage 8→4→2→1 reduction tree.

Test 3.3: Maximum positive magnitude. Test objective: Stress the INT4 array at maximum positive nibble value. Test procedure: A=B=0x77777777 (eight nibbles of +7). Expected outcome: accumulator[31:0] = 8 · 49 = 392.

Test 3.4: Maximum negative magnitude. Test objective: Stress the INT4 array at minimum negative nibble value. Test procedure: A=B=0x88888888 (eight nibbles of −8). Expected outcome: accumulator[31:0] = 8 · 64 = 512. Confirms correct sign treatment at the asymmetric INT4 boundary.

Test 3.5: Mixed signs. Test objective: Verify sign correctness across alternating-sign nibbles. Test procedure: A=0x78787878, B=0x87878787. Expected outcome: accumulator[31:0] equals golden_mac4 of these inputs.

Test 3.6: Sequential accumulation. Test objective: Verify the INT4 accumulation feedback path. Test procedure: Eight consecutive identity MAC4 operations. Expected outcome: accumulator[31:0] = 64.

Group 4: ADD_BIAS Directed Tests

Test 4.1: Zero bias. Test objective: Verify that a zero bias does not perturb the accumulator. Test procedure: Issue ADD_BIAS with port_B = 0 from a zeroed accumulator. Expected outcome: accumulator[31:0] = 0.

Test 4.2: Repeated unit bias. Test objective: Verify that repeated single-unit bias additions accumulate linearly. Test procedure: Issue 100 consecutive ADD_BIAS operations with port_B = 1. Expected outcome: accumulator[31:0] = 100. Confirms the bias path correctly drives acc_source = 2'b11 and that no spurious feedback corruption occurs across many sequential operations.

Test 4.3: Most-negative bias. Test objective: Verify correct handling of port_B = 0x80000000 (the asymmetric two's-complement minimum). Test procedure: Issue ADD_BIAS with port_B = 0x80000000 from zero. Expected outcome: accumulator[31:0] = −2147483648. Confirms 32-bit signed addition at the negative boundary.

Test 4.4: Overflow wrap. Test objective: Verify two's-complement wrap behavior on overflow, consistent with the 32-bit lower adder semantics. Test procedure: Preload accumulator with 0x7FFFFFFF via ADD_BIAS, then add +1. Expected outcome: accumulator[31:0] = 0x80000000. Confirms the lower adder wraps cleanly without saturation, matching the architectural specification.

Test 4.5: Signed mixed bias. Test objective: Verify cancellation between positive and negative bias contributions. Test procedure: ADD_BIAS +1,000,000 followed by ADD_BIAS −500,000. Expected outcome: accumulator[31:0] = +500,000.

Group 5: SCALE Directed Tests

This phase is the centerpiece of the unit verification, since the SCALE operation involves the only multi-cycle datapath in the module: byte broadcast, holding register capture on cycle 0, stitching tree with sign-aware extension on cycle 3, alignment shift, gated upper adder, and full 64-bit accumulator capture.

Test 5.1: Zero accumulator. Test objective: Verify that SCALE on a zeroed accumulator produces zero, regardless of the scaling factor. Test procedure: Reset, then issue SCALE with port_B = 0xCAFEBABE. Expected outcome: accumulator = 64'b0. This implicitly confirms that acc_load on cycle 0 correctly zeros the upper 32 bits, since any leakage would produce a non-zero result.

Test 5.2: Unity. Test objective: Verify the simplest non-trivial scaling. Test procedure: Preload accumulator = 1 via ADD_BIAS, then SCALE with port_B = 1. Expected outcome: accumulator = 64'd1.

Test 5.3: Maximum unsigned scaling factor. Test objective: Stress the stitching tree by saturating port_B. Test procedure: Preload accumulator = 1, SCALE with port_B = 0xFFFFFFFF. Expected outcome: accumulator = 64'h00000000_FFFFFFFF. Confirms that all four byte products and their cross-position carries propagate correctly.

Test 5.4: Negative accumulator with maximum scaling factor. Test objective: Verify the cycle-3 sign-extension path. Test procedure: Preload accumulator = 0xFFFFFFFF (= −1 signed), SCALE with port_B = 0xFFFFFFFF. Expected outcome: accumulator = 64'hFFFFFFFF_00000001 (= −0xFFFFFFFF). On cycle 3, the high byte (0xFF) must be treated as the signed sign byte, producing a negative 40-bit partial product that propagates correctly through the upper 32-bit adder.

Test 5.5: Maximum positive × maximum unsigned. Test objective: Verify the largest positive product the module can produce. Test procedure: Preload accumulator = 0x7FFFFFFF, SCALE with port_B = 0xFFFFFFFF. Expected outcome: accumulator = golden_scale(0x7FFFFFFF, 0xFFFFFFFF) = 0x7FFFFFFE_80000001.

Test 5.6: Maximum negative × maximum unsigned. Test objective: Verify the most-negative product the module can produce — the asymmetric corner. Test procedure: Preload accumulator = 0x80000000 (= −2³¹), SCALE with port_B = 0xFFFFFFFF. Expected outcome: accumulator = golden_scale(0x80000000, 0xFFFFFFFF) = 0x80000000_80000000. This is the worst case for both the cycle-3 sign-extension width and the upper adder's signed-arithmetic correctness.

Test 5.7: Carry propagation stress. Test objective: Force carry propagation across every byte-pair boundary in the stitching tree and across the lower-to-upper adder boundary in the accumulation unit. Test procedure: Preload accumulator = 0x01010101, SCALE with port_B = 0xFFFFFFFF. Each byte product is 0xFF, and stitching produces a long chain of carries. Expected outcome: accumulator equals golden_scale of these inputs. Any dropped carry between the 24-bit and 40-bit stitch stages, or between the lower and upper accumulators, will produce a one-bit error visible in the final result.

Test 5.8: Distinct-byte holding register correctness. Test objective: Confirm that bytes 1, 2, and 3 of the original accumulator are correctly preserved through the holding register across cycles 1, 2, and 3 of the SCALE sequence. Test procedure: Preload accumulator = 0xA1B2C3D4 (four distinct nonzero bytes), SCALE with port_B = 0x11223344. Expected outcome: accumulator equals golden_scale(0xA1B2C3D4, 0x11223344). Since cycle 0 overwrites the original accumulator, any byte misrouting in the holding register or broadcast mux would produce a wrong contribution on at least one of the subsequent cycles, resulting in a measurable error.

Group 6: Operand Isolation Tests

These tests directly verify the operand-isolation gates that sit at the entrances of the three reduction paths. They drive synthetic stimulus that the production Controller never produces, in order to isolate the gates themselves from their normal control envelope.

Test 6.1: MAC8 path isolation. Test objective: Verify that with mac8_iso_en = 0, the MAC8 reduction tree contributes zero regardless of the multiplier outputs. Test procedure: Preload accumulator = 100 via ADD_BIAS. Drive MAC8-shaped controls (mode4x4 = 0, sign flags high, acc_source = 2'b01, acc_wr_en = 1) with both mac4_iso_en and mac8_iso_en deasserted, while presenting non-zero operands on port_A and port_B. Expected outcome: accumulator[31:0] = 100. The MAC8 path is isolated, the addend is zero, and the accumulator is unchanged.

Test 6.2: MAC4 path isolation. Test objective: Verify the symmetric isolation behavior for the MAC4 reduction tree. Test procedure: Preload accumulator = 200, drive MAC4-shaped controls with both mac iso enables low. Expected outcome: accumulator[31:0] = 200.

Test 6.3: SCALE path isolation. Test objective: Verify that with scale_iso_en = 0, the stitching tree contributes zero on every cycle of the SCALE sequence. Test procedure: Preload accumulator = 300. Manually drive the four-cycle SCALE waveform (acc_load on cycle 0, scale_phase walking 00→11) with scale_iso_en held low across all four cycles. Expected outcome: accumulator = 64'b0. Cycle 0 acc_load zeros the feedback and loads a zero addend; cycles 1–3 add zero addends to the running zero. Any non-zero result indicates an isolation gate failure.

Group 7: Mixed Sequence Tests

Test 7.1: Two MAC8 followed by SCALE. Test objective: Verify that SCALE correctly consumes the running MAC accumulator and produces the expected 64-bit product. Test procedure: Issue two MAC8 operations with arbitrary signed operands, then SCALE with port_B = 0x10. Expected outcome: accumulator equals golden_scale of (sum of two MAC8 results) and 0x10. Validates the boundary between MAC's 32-bit accumulation regime and SCALE's 64-bit regime.

Test 7.2: MAC4 followed by SCALE. Test objective: Verify the same boundary in INT4 mode. Test procedure: One MAC4 followed by SCALE with port_B = 0x0F. Expected outcome: accumulator equals golden_scale of the MAC4 result and 0x0F.

Test 7.3: ADD_BIAS followed by SCALE. Test objective: Verify the typical pre-SCALE flow used in real inference workloads. Test procedure: ADD_BIAS with arbitrary signed bias, then SCALE. Expected outcome: accumulator equals golden_scale(bias, port_B).

Test 7.3: Back-to-Back SCALE sequences. Test objective: Verify that a second SCALE is uncorrupted by residual state from the first. Test procedure: Issue SCALE #1, then RST_ACC, ADD_BIAS, and SCALE #2. Expected outcome: Both results match their respective golden_scale values. A failure here indicates stale holding register data from SCALE #1 contaminating SCALE #2.

Test 7.4: Mid-Stream RST_ACC. Test objective: Verify that rst_acc cleanly discards prior MAC results without affecting subsequent operations. Test procedure: Two MAC8 operations, RST_ACC, one final MAC8. Expected outcome: accumulator[31:0] equals only the final MAC8 result.

Group 8: Constrained Random Verification

Test 8: 10,000-cycle random stimulus.

Test objective: Validate accumulator correctness across long, unpredictable mixes of MAC4, MAC8, ADD_BIAS, and SCALE operations, with state propagated continuously across iterations. Expected outcome: All 20 output comparisons pass on every cycle. No structural invariant violations occur. This validates the Controller's resilience to arbitrary input sequences including mid-sequence configuration changes, chp_slct toggling during and between instructions, invalid encodings mixed with valid ones, and the FSM's immunity to input noise during multi-cycle operation.

Test procedure: Over 10,000 iterations, the testbench randomly selects one of four operation profiles with equal probability. Operands are drawn from a boundary-biased generator: with 60% probability each operand is a uniformly random 32-bit value, and with 40% probability each byte is independently selected from the boundary set {0x00, 0x01, 0x7F, 0x80, 0xFF, fully-random}. Boundary biasing concentrates stimulus near the asymmetric two's-complement edges and the unsigned-vs-signed crossover that historically produce sign-extension and carry-chain bugs in arithmetic datapaths.

A software shadow accumulator is updated by the appropriate golden function on every iteration: shadow_lo += golden_mac8/4 for MAC operations, shadow_lo += signed(port_B) for ADD_BIAS, and shadow ← golden_scale(shadow_lo, port_B) for SCALE (with shadow_lo subsequently aliased to shadow[31:0] for the next iteration). After every operation, the testbench compares shadow against the hardware accumulator and logs any divergence with the seed and operation index.

Expected outcome: zero divergences across all 10,000 iterations. Because the shadow propagates continuously, any state-leakage bug — stale holding register contents, upper-32 contamination during MAC, mis-applied acc_load, dropped carry between the lower and upper adders — compounds across iterations and surfaces as soon as it reaches an observed bit. This is the strongest end-to-end validation of the accumulation unit and the SCALE multi-cycle datapath.