PE Sanity Test Plan

Black-Box Verification Gate Prior to FPGA Platform Handoff

# 1. Scope and Objectives

This document defines the verification plan for the Processing Element (PE) as a black-box system, exercised exclusively through its top-level port list as specified in the PE Definition Document. Sub-module-level concerns (arithmetic correctness of the multiplier array, FSM transition coverage of the Controller, byte-mux isolation in the I/O Block, etc.) are out of scope; each was closed by its respective unit testbench prior to integration.

The plan is a sanity gate: its purpose is to demonstrate that the integrated PE is wired correctly, honors its external protocol, and produces SV-equivalent results for a representative set of neural-network micro-sequences in both INT8 and INT4 precision modes. Exhaustive randomized stress and platform-level timing validation are deferred to the FPGA verification effort.

## 1.1 Verification Goals

- Confirm correct top-level integration of the Controller, MAC & SCALE, PPU, and I/O Block sub-modules through observable behavior at the external interface.

- Demonstrate compliance with the external protocol defined in the PE Definition Document — instruction sampling rules, validity gating, chip-select gating, pe_ready flow control, and output_valid timing.

- Establish end-to-end golden agreement with a SV reference model for full inference micro-sequences across both precision modes and all eight activation functions.

- Surface integration-class defects (incorrect mux precedence, missing isolation gating, wrong control-signal fan-out) that unit-level testing cannot expose by construction.

## 1.2 Non-Goals

- Exhaustive arithmetic stress of the MAC tree, requantizer shifter, or activation LUTs. These were verified at the unit level via constrained random and directed boundary testing.

- Post-synthesis or gate-level simulation. The plan targets the synthesizable RTL only; STA closure is handled in the synthesis flow.

- Multi-PE system-level behavior. The PE is the unit of integration; system topology is outside this document.

- High-volume randomized regression. A light random smoke layer is included to surface unexpected sequencing interactions; deep random soak is the FPGA team's responsibility on their platform.

# 2. Verification Environment

The environment follows the structure described in the Sub-Modules General Verification Methodology document, scaled to the PE boundary. The DUT is the PE top-level module instantiated as uut, with all external ports driven by the testbench.

## 2.1 Architecture

- DUT: PE top-level. Only the eleven external ports — clk, rst, chp_slct, valid_opcode, pe_opcode, valid_A, valid_B, operand_A, operand_B, data_out, pe_ready, valid_output — are accessible to the testbench. No internal hierarchical references are used.

- Stimulus generation: A SystemVerilog driver implementing one task per opcode (drive_nop, drive_rst_acc, drive_mac, drive_add_bias, drive_scale, drive_load_cfg, drive_exec_pp, drive_read_acc_byte, drive_read_cfg). Each task asserts the required validity signals, drives the opcode and operands for one cycle, and de-asserts them on the following edge. A wait_ready helper blocks until pe_ready returns high after multi-cycle operations.

- Reference model: A SV implementation of the PE's instruction semantics. The model maintains the four user-visible state variables — configuration register, accumulator, shift-amount register, and output register — and a 5-bit pending opcode. For each driven instruction, the model is advanced and the predicted value of any output-producing signal is recorded for later comparison. INT8 and INT4 arithmetic are performed using SV int8/int16/int64 dtypes; SCALE uses an unsigned 32-bit ×  signed-low-32-bit-of-accumulator computation that matches the hardware's stitching tree exactly.

- Result evaluation: A check_dout task triggered on each valid_output rising edge. It compares data_out against the reference model using ===. For accumulator state, an eight-byte readout sequence (READ_ACC_BYTE × 8 with operand_A[2:0] = 0…7) reconstructs the 64-bit value and the result is compared against the model accumulator. Pass/fail counts are accumulated per test group and a summary is printed at simulation end.

- Coverage: Functional coverage is collected on (opcode, mode4x4) pairs and on each activation function. Code coverage (line, toggle, FSM state) is collected during the full regression run and exported for the project report.

## 2.2 Clock and Reset

A 100 MHz clock is generated by the testbench (10 ns period). Reset is asserted asynchronously for at least three clock periods at simulation start and is also exercised mid-simulation by Test Group G1. All stimulus is driven combinationally and sampled on the rising clock edge by the DUT, mirroring the production scenario.

Internal resets are routed through a 2-stage Asynchronous Assert, Synchronous Deassert (AASD) synchronizer.

# 3. Golden Reference Model

The reference model is a cycle-accurate functional model implemented as a native SystemVerilog class (pe_predictor) embedded in the testbench. It is not a structural twin of the RTL — it mirrors the architectural state changes specified in the PE Definition Document, one instruction at a time, with no notion of multi-cycle SCALE phase: the four SCALE execution cycles are collapsed into a single atomic state update at the point the SCALE instruction is committed.

## 3.1 State Variables

| State | Width | Reset Value | Updated By |
| --- | --- | --- | --- |
| accumulator | 64-bit signed | 0 | RST_ACC, MAC, ADD_BIAS, SCALE |
| config_reg | 4-bit | 4'b0000 | LOAD_CFG |
| shamt_reg | 6-bit | 6'b0 | SCALE (cycle 0 capture) |
| data_out_reg | 8-bit | 8'b0 | EXEC_PP, READ_ACC_BYTE, READ_CFG |

## 3.2 Reference Arithmetic

- INT8 MAC: Interpret port_A and port_B as four signed int8 elements each. Compute the dot product, then accumulate into the 64-bit accumulator with two's-complement wrap.

- INT4 MAC: Interpret each operand byte as two packed signed int4 elements (eight per operand). Compute the dot product, accumulate with two's-complement wrap.

- ADD_BIAS: Sign-extend operand_B to 64 bits and add to the accumulator with two's-complement wrap.

- SCALE: Compute (accumulator[31:0] interpreted as int32 two's-complement) × (operand_B as uint32). The 64-bit result overwrites the accumulator. The lower six bits of operand_A are captured into shamt_reg.

- EXEC_PP: Arithmetic right-shift the accumulator by shamt_reg with round-to-nearest-integer. Saturate to the signed range of the active precision mode ([-128, 127] for INT8, [-8, 7] for INT4). Apply the activation function selected by config_reg[2:0]. The 8-bit result (sign-extended in INT4 mode) overwrites data_out_reg.

The activation function reference is a Python dictionary mapping the eight encodings to their implementations. Linear functions (Identity, ReLU, Leaky ReLU ×2) are computed directly. Non-linear functions (Tanh, Sigmoid, GELU, SiLU) are computed from the quantized LUT entries in the Activation Block RTL via a one-time Python-side build that mirrors the case-statement contents. This guarantees that any RTL-LUT discrepancy is caught as a planned outcome, not a model error.

# 4. Test Groups

Tests are organized into ten groups, ordered to surface foundational failures before high-level ones. Each group declares an objective, a procedure, an expected outcome, and a pass criterion. A group fails the regression if any test case within it fails.

## G1. Reset and Initialization

Objective. Confirm that asynchronous reset establishes a fully defined idle state across all sub-modules and that reset dominates any concurrent input activity.

### G1.1 Power-On Reset

Drive rst = 0 from time zero, then release after three clock periods with all other inputs at zero. Verify pe_ready = 1, valid_output = 0, and data_out = 8'h00 two clock cycles after rst goes high.

### G1.2 Mid-Operation Reset

Load a non-default configuration (LOAD_CFG with INT8 + GELU), execute several MACs and an ADD_BIAS to leave a non-zero accumulator, then drop rst asynchronously between clock edges. Following reset release, issue READ_CFG and verify data_out = 8'h00. Issue READ_ACC_BYTE for all eight indices and verify each returns zero. This proves the configuration register, accumulator, output register, and shift-amount register all clear correctly.

### G1.3 Reset Dominance

Hold rst = 0 while driving random combinations of pe_opcode, operands, and validity signals for ten cycles. Verify that pe_ready remains high, valid_output remains low, and data_out remains zero throughout. Reset must dominate every input path.

## G2. External Interface Gating

Objective. Confirm that chp_slct, valid_opcode, valid_A, and valid_B each gate their respective capture paths exactly as specified in the PE Definition Document. Integration class: misrouting of any validity signal would produce false captures or missed captures that no unit test can detect.

### G2.1 chp_slct Low

With pe_ready = 1 and all validity signals high, drive a valid LOAD_CFG with operand_A[3:0] = 4'b1111 while chp_slct = 0. Follow with a READ_CFG (chp_slct = 1) and verify data_out = 8'h00, confirming the LOAD_CFG was rejected at the boundary.

### G2.2 Validity Signal Independence

Drive a MAC opcode with only valid_A = 1 (valid_B = 0). The opcode should be captured but operand_B_reg should retain its prior value. Cross-check by capturing operand_B in a preceding cycle to a known sentinel and verifying the MAC result reflects the sentinel, not the value present on the operand_B bus during the MAC cycle.

### G2.3 Opcode Validity

With valid_opcode = 0 and chp_slct = 1, drive each of the nine valid opcodes for one cycle each. Verify no observable state change (READ_CFG and READ_ACC_BYTE return their prior values; no spurious valid_output).

### G2.4 Invalid Opcode Mapping

Drive each of the 23 non-allocated 5-bit encodings (all values not in {0..8}) with valid_opcode = 1 and chp_slct = 1. Verify that none cause observable state changes — they must all decode as NOP per the Controller's opcode validator.

## G3. Configuration Register

Objective. Validate the LOAD_CFG → READ_CFG round trip across the encoding space and confirm that the configuration register is the sole driver of mode4x4 and act_fn_sel observed via downstream behavior.

### G3.1 Encoding Round-Trip

For each of the 16 valid configuration values (one per row of the configuration register encoding table), issue LOAD_CFG with operand_A[3:0] = cfg, follow with READ_CFG, and verify data_out == {4'b0, cfg}. This closes the loop on both the LOAD_CFG capture path and the READ_CFG observation path.

### G3.2 Persistence

After loading a non-default configuration, issue a long sequence of unrelated instructions (RST_ACC, MAC, ADD_BIAS, READ_ACC_BYTE × 8). Re-issue READ_CFG and verify the original configuration is intact. This proves that the configuration register is updated only by LOAD_CFG.

## G4. Accumulator Direct Manipulation

Objective. Establish observability of the 64-bit accumulator via READ_ACC_BYTE and validate the three instructions that touch the accumulator without engaging the multiplier array: RST_ACC, ADD_BIAS, and the byte-readout path itself.

### G4.1 RST_ACC Establishes Zero

Issue RST_ACC. Read all eight accumulator bytes via READ_ACC_BYTE with operand_A[2:0] sweeping 0 through 7. Each byte must read as 8'h00.

### G4.2 ADD_BIAS Single-Shot

Drive ADD_BIAS with a representative set of operand_B values: 32'h0000_0001 (smallest positive), 32'hFFFF_FFFF (-1), 32'h7FFF_FFFF (largest positive int32), 32'h8000_0000 (most negative int32), and one mid-range positive value. After each, read all eight accumulator bytes and reconstruct the 64-bit value. Compare against the reference model, which sign-extends operand_B to 64 bits.

### G4.3 Byte-Select Index Coverage

After an ADD_BIAS that loads a distinct value into each of the eight bytes of the accumulator (e.g., operand_B sign-extended from a pattern like 64'h0807_0605_0403_0201), read each byte via the corresponding operand_A[2:0] index. The eight reads must match the eight bytes in order. This validates the byte-select multiplexer path and the operand_A_reg[2:0] → mux-select wiring.

## G5. MAC Datapath

Objective. Confirm correct MAC behavior in both precision modes via the accumulator readout path. Sanity-level only — exhaustive coverage of the multiplier array is closed by the MP_Multiplier and MAC_and_SCALE unit testbenches.

### G5.1 INT8 Directed Vectors

LOAD_CFG with config_reg[3] = 1 (INT8). Issue RST_ACC, then sequence the following MAC operations with eight-byte readouts in between, comparing each against the reference:

- MAC with operand_A = 0, operand_B = arbitrary. Expected: accumulator unchanged.

- MAC with all four lanes maximally positive (operand_A = operand_B = 32'h7F7F_7F7F). Expected: 4 × 127² = 64,516.

- MAC with all four lanes maximally negative (operand_A = operand_B = 32'h8080_8080). Expected: 4 × 128² = 65,536.

- MAC with mixed-sign lanes (operand_A = 32'h7F7F_8080, operand_B = 32'h80807F7F). Expected: reference-computed sum of four signed 16-bit products.

- Eight consecutive MACs of all-ones bytes (operand_A = operand_B = 32'h0101_0101). Expected: accumulator = 32, confirming accumulation with no premature reset.

### G5.2 INT4 Directed Vectors

LOAD_CFG with config_reg[3] = 0 (INT4). Each operand byte is two packed int4 elements. Issue RST_ACC, then run the analogous sequence:

- MAC with operand_A = 0. Expected: accumulator unchanged.

- MAC with all eight INT4 lanes = +1 (operand_A = operand_B = 32'h1111_1111). Expected: 8 × 1 = 8.

- MAC with all eight INT4 lanes maximally positive (operand_A = operand_B = 32'h7777_7777). Expected: 8 × 49 = 392.

- MAC with all eight INT4 lanes maximally negative (operand_A = operand_B = 32'h8888_8888). Expected: 8 × 64 = 512.

- MAC with mixed-sign INT4 vectors. Expected: reference-computed sum.

### G5.3 Mode-Switch Smoke

Verify that mode4x4 is sourced from the configuration register on a cycle-to-cycle basis: load INT8 mode, MAC once, load INT4 mode, MAC once with the same operand_A/operand_B values. The two reads must differ as predicted by the reference. This proves the configuration register's mode bit is correctly routed through the Controller to the MAC & SCALE.

## G6. SCALE Multi-Cycle Operation

Objective. Validate the SCALE multi-cycle protocol: pe_ready behavior, shift-amount register loading, instruction rejection during SCALE execution, and final-cycle accumulator overwrite. The SCALE instruction is the only place in the PE where the external protocol diverges from single-cycle behavior; the bulk of this group targets that divergence.

### G6.1 pe_ready Profile

With the FSM in IDLE, sample pe_ready = 1. Issue SCALE on cycle T (valid_opcode, valid_A, valid_B, chp_slct all high). On cycle T+1 sample pe_ready and verify it has dropped to 0 (Controller deasserts on the same cycle SCALE is decoded). Sample pe_ready on cycles T+2, T+3, and T+4 and verify it remains 0. Sample on cycle T+5 and verify it has returned to 1. This profile confirms the FSM transitions IDLE → SC1 → SC2 → SC3 → IDLE and that pe_ready is correctly derived from the FSM state and opcode register.

### G6.2 Instruction Rejection During SCALE

After issuing SCALE, attempt to issue a MAC on each of cycles T+1 and T+2 (states SC1 and SC2) with valid_opcode and chp_slct asserted. Following the SCALE completion, read back the accumulator and verify the value matches a SCALE-only reference. The injected MACs must have been rejected because pe_ready was low; any leakage would corrupt the accumulator and the comparison would fail.

### G6.3 Directed SCALE Vectors

LOAD_CFG to a known mode. For each vector below, preload the accumulator via ADD_BIAS, issue SCALE, wait for pe_ready, then read all eight accumulator bytes and compare against the reference. Shift amount is supplied via operand_A[5:0]; scale factor (treated as unsigned 32-bit) via operand_B.

- Identity scale: accumulator preset to 32'h0000_0001 (lower 32 bits), shamt = 0, scale = 32'h0000_0001. Expected accumulator = 64'h0000_0000_0000_0001.

- Pure-positive scale: accumulator lower 32 = 32'h0000_1000, scale = 32'h0000_1000. Expected = 32'h0000_1000 × 32'h0000_1000 = 64'h0000_0000_0100_0000.

- Negative low-word: accumulator lower 32 = 32'hFFFF_FFFF (-1 as int32), scale = 32'h0000_0001 (1 as uint32). Expected = sign-extended -1, then × 1 = 64'hFFFF_FFFF_FFFF_FFFF.

- Large unsigned scale: accumulator lower 32 = 32'h0000_0002, scale = 32'hFFFF_FFFF. Expected = 2 × (2³² − 1), 64-bit.

- Most-negative low-word: accumulator lower 32 = 32'h8000_0000, scale = 32'h0000_0002. Expected = (−2³¹) × 2 = −2³², which is representable in 64 bits.

### G6.4 shamt Persistence

Issue SCALE with operand_A[5:0] = 6'd31. Following SCALE completion, run several unrelated instructions (RST_ACC, MAC, ADD_BIAS). Then load the accumulator to a known value, issue EXEC_PP, and verify data_out matches a reference computed with shamt = 31. This proves the shift-amount register holds across non-SCALE instructions and is only re-loaded by SCALE.

## G7. Activation Function Sweep

Objective. Exercise each (precision mode, activation function) combination through the full EXEC_PP path at least three times — once with a small-magnitude input, once with a value near the positive saturation boundary, and once with a value near the negative saturation boundary. Sixteen pairs × three samples = 48 EXEC_PP invocations. Per-activation exhaustive coverage is the Activation Block unit testbench's responsibility; here we verify routing only.

### G7.1 Procedure

For each (mode4x4, act_fn_sel) pair:

- LOAD_CFG with the corresponding 4-bit configuration.

- RST_ACC.

- ADD_BIAS to load a known 64-bit value into the accumulator.

- SCALE with a chosen shamt and scale factor of 1 to install the shift amount without disturbing the desired pre-PPU value (preload the accumulator after SCALE if needed).

- EXEC_PP. On valid_output rising edge, capture data_out and compare against the reference.

### G7.2 Pass Criterion

All 48 invocations must match the reference exactly. The principal failure mode this group catches is mis-routing of act_fn_sel from the configuration register to the Activation Block (e.g., a stale or inverted bit), which a unit test on the Activation Block cannot detect.

## G8. End-to-End Inference (Golden Agreement)

Objective. This group is the gate. It exercises the PE through the full instruction sequence of a neural-network neuron — RST_ACC, a series of MACs, ADD_BIAS, SCALE, EXEC_PP — and demands bit-exact agreement with SystemVerilog golden model. All previous groups exist to make a failure here debuggable; passing G8 in both precision modes across all activations is the necessary and sufficient condition for handoff to the FPGA team.

### G8.1 Single-Neuron INT8 Sequence

Procedure: LOAD_CFG with INT8 and a chosen activation. RST_ACC. Issue N MAC instructions (with N ∈ {1, 4, 8, 16}) using directed but representative operand pairs (mixed signs, varying magnitudes, no all-zeros). Issue ADD_BIAS with a representative bias. Issue SCALE with a representative shamt and scale factor. Issue EXEC_PP. Compare data_out against the SystemVerilog golden model.

### G8.2 Single-Neuron INT4 Sequence

Same procedure as G8.1 but with INT4 mode. N ∈ {1, 8, 16, 32}. Verify data_out[3:0] matches; data_out[7:4] is don't-care per the PE Definition Document.

### G8.3 Activation Sweep at the End-to-End Level

Repeat G8.1 and G8.2 with each of the eight activation functions selected. This yields 16 end-to-end micro-sequences, each fully golden-checked.

### G8.4 Sequential-Layer Chaining

Drive a two-stage sequence: compute a neuron in INT8 with Tanh, capture data_out via valid_output, then immediately reuse the captured value as one byte of operand_A in a second neuron's MAC stream. The second neuron's data_out is compared against a two-stage SystemVerilog golden model. This proves the data_out path is not only correct but also stable enough across cycles to participate in a layer-to-layer dataflow.

### G8.5 Mode Transition Mid-Sequence

Compute one neuron in INT8, then issue LOAD_CFG to switch to INT4, then compute a second neuron in INT4. Both data_out values must match the reference. This catches any latent state that would cause cross-mode contamination.

## G9. Protocol Assertions (Always-On Monitors)

Objective. These are not test cases — they are SystemVerilog concurrent assertions that run for the duration of every other test group. Any violation fails the regression regardless of which group was active when it fired. They encode invariants of the PE's external contract.

| ID | Invariant |
| --- | --- |
| A1 | valid_output asserts for exactly one clock cycle (no multi-cycle stretches). |
| A2 | valid_output may assert 1 or 2 cycles after a cycle in which EXEC_PP, READ_ACC_BYTE, or READ_CFG was the validated command. |
| A3 | pe_ready is low if and only if the SCALE FSM is in SC1, SC2, SC3, or the cycle SCALE is decoded in IDLE (observable as a 3 to 5 low window following SCALE). |
| A4 | data_out and valid_output are stable (not X) at all times after the first three cycles post-reset release. |
| A5 | While rst is low, pe_ready is high and valid_output and data_out are low. |
| A6 | never asserts if chp_slct was low for the prior two consecutive cycles and no instruction was already in flight. |

Assertions are written as SystemVerilog properties bound to the testbench module. A1, A2 and A3 in particular catch the two highest-risk integration bugs: a stuck output_valid driver, and a pe_ready that drops on the wrong instruction class.

## G10. Lightly Randomized Smoke

Objective. Surface unforeseen sequencing interactions that the directed groups did not anticipate. The smoke layer is intentionally bounded — a few thousand cycles, not millions — because deep randomized soak is the FPGA team's domain on their platform.

### G10.1 Procedure

Run 2,000 randomly-generated instructions through the same reference-checked driver used in G8. The randomizer biases its opcode distribution toward MAC and ADD_BIAS (60% combined), interleaves SCALE (8%), EXEC_PP (8%), READ_ACC_BYTE (10%), READ_CFG (4%), RST_ACC (4%), LOAD_CFG (4%), and NOP (2%). Operands and configuration values are uniformly random. chp_slct, valid_opcode, valid_A, and valid_B are each independently held with 90% probability.

### G10.2 Pass Criterion

Every output-producing instruction (every valid_output pulse) must match the reference. Every protocol assertion from G9 must hold for the entire run. Failure on either count is debugged to root cause before handoff; the smoke layer is not allowed to pass with 'known minor mismatches.'

# 5. Exit Criteria — FPGA Handoff Gate

The PE is cleared for FPGA platform stress testing when, and only when, all of the following hold simultaneously on a single clean regression run:

- Every test case in G1 through G8 reports PASS.

- Every assertion in G9 holds for the full regression duration.

- G10 reports zero data_out mismatches across the 2,000-instruction smoke run.

- Functional coverage shows 100% of (opcode, mode4x4) pairs and 100% of activation functions exercised at least once with a passing outcome.

- Line coverage on the integrated PE RTL is ≥ 95%.

On meeting these conditions, the RTL bundle (PE.v and all sub-module sources), the testbench, the SystemVerilog golden model, the regression log, and the coverage report are packaged for the FPGA team. Any deviation found by the FPGA stress run is fed back as a new directed test case in the appropriate group above; the sanity regression is re-run before resubmission.

# 6. Out of Scope

The following are explicitly deferred to other verification activities and must not be relied upon as conditions of this gate:

- Gate-level simulation with back-annotated SDF — covered by the post-synthesis flow.

- Static timing analysis closure at the target clock — covered by the synthesis and place-and-route reports.

- Multi-million-cycle randomized stress — covered by the FPGA team's platform regression.

- X-propagation analysis under partial-reset conditions — covered by a separate formal property check.

- Power-aware simulation — covered by the synthesis power analysis flow.